UNITED STATES DEPARTMENT OF THE INTERIOR

GEOLOGICAL SURVEY

Gold, mercury, tellurium, and thallium data of

stream-sediment samples from the Iditarod quadrangle, Alaska

By

D.M. Hopkins, J.E. Gray, P.L. Hageman,

C.M. McDougal, and K.E. Slaughter

Open-File Report 91-283-B Diskette version

This report is preliminary and has not been reviewed for

conformity with U.S. Geological Survey editorial standards and

stratigraphic nomenclature.  Any use of trade names is for

descriptive purposes only and does not imply endorsement by the

USGS.

1991

CONTENTS

Page

STUDIES RELATED TO AMRAP ..................................... 1

INTRODUCTION ................................................. 1

GENERAL GEOLOGY .............................................. 2

METHODS OF STUDY ............................................. 2

Sample Media ........................................... 2

Sample Collection ...................................... 3

Sample Preparation ..................................... 3

Sample Analysis ........................................ 3

DATA STORAGE SYSTEM .......................................... 4

DESCRIPTION OF THE STATPAC FILE .............................. 5

REFERENCES CITED ............................................. 6

TABLES

Table 1.  Lower limits of determination for gold, mercury,

tellurium, and thallium in stream-sediment

analysis ........................................... 4

Table 2.  Comparison of gold analyses of GXR standards using

atomic absorption spectroscopy ..................... 4

OTHER FILES

DATASS.STP  A STATPAC file containing gold, mercury, tellurium,

and thallium data for stream-sediment samples from

the Iditarod quadrangle, Alaska

README      An ASCII file describing the diskette version of

this report

STP2DAT.EXE  A conversion program providing format options for

transforming the DATASS.STP file

TEXT.DOC    The ASCII file of the text portion of this report

-i-

STUDIES RELATED TO AMRAP

The U.S. Geological Survey is required by the Alaska

National Interests Lands Conservation Act (Public Law 96-487,

1980) to survey certain federal lands to determine their mineral

potential.  Results from the Alaska Mineral Resource Assessment

Program (AMRAP) must be made available to the public and be

submitted to the President and Congress.  This report is one of a

series of publications that presents geochemical and

mineralogical results collected from the mineral assessment study

of the Iditarod quadrangle, Alaska.

INTRODUCTION

During the summers of 1984-86, a reconnaissance geochemical

survey was conducted in the Iditarod quadrangle, Alaska.

A large geochemical data base for the quadrangle was published in

Gray and others (1988).  The gold (Au), mercury (Hg), tellurium

(Te), and thallium (Tl) data for stream-sediment samples

presented here are a supplement to this geochemical data base and

are used to assist exploration for mineral occurrences in the

quadrangle.  A subset of 240 stream-sediment samples were

analyzed for Te to evaluate its usefulness as a geochemical

pathfinder in the study area.  The samples analyzed were those

with the most potential to assist in the mineral assessment

studies of the quadrangle.  These Te results were not favorable

because only two stream sediment samples contained concentrations

above the 0.05 ppm lower limit of determination.  Therefore,

additional analyses for Te were discontinued in this study.

The Iditarod quadrangle is bounded by 62 to 63 degrees

north latitude and by 156 to 159 degrees west longitude. The area

comprises approximately 6,700 square miles (17,350 square

kilometers) in the west-central portion of the Alaskan

interior.  The quadrangle includes the Beaver Mountains and a

portion the Kuskokwim Mountains.  Part of the Innoko National

Wildlife Refuge is located in the northwestern corner of the

quadrangle and is also included in the study area.  The quadrangle

is sparsely populated with two small communities at Flat and

Takotna and a few isolated mining camps.  Few roads exist

throughout the quadrangle and access to much of the area is limited

to travel by air or foot.  Boat access is possible on some of the

larger rivers.

The terrain is dominated by low rolling hills and broad

sediment filled lowlands.  This terrain is best exemplified by

the Kuskokwim Mountains in the central portion of the quadrangle.

The most rugged topographic expression occurs in the Beaver

Mountains and a few other mountain peaks scattered throughout the

quadrangle.  The maximum elevation in the quadrangle is 4055 feet

(1236 meters) and is located in the northern Beaver Mountains.

Much of the western portion of the quadrangle is swampy,

especially in the Yetna and Iditarod River basins.  Most of the

quadrangle is covered with vegetation that ranges from northern

latitude forests to subarctic tundra.

-1-

GENERAL GEOLOGY

Cretaceous sedimentary rocks of the Kuskokwim Group form

the dominant bedrock in the Iditarod quadrangle (Cady and others,

1955).  These rocks consist of thick sequences of intercalated

sandstones, shales, and conglomerates (Bundtzen and Laird, 1983).

Rocks of the Kuskokwim Group primarily represent deep water

turbidite facies, but small amounts of shallow shoreline facies

rocks also occur in the sequences (Miller and Bundtzen, 1987).

These rocks have been deformed into northeast trending synclines

and anticlines; high-angle faults appear to parallel these

folds.  A major northeast trending strike-slip fault, the

Iditarod-Nixon Fork fault, transects the central portion of the

quadrangle.

Late Cretaceous to early Tertiary volcano-plutonic

complexes intrude or overlie the Kuskokwim sedimentary rocks at

several localities.  These complexes consist of basalt and

andesite volcanic flows that are in fault contact with or overlie

monzonite plutons.  An extensive felsic to mafic volcanic field,

that is coeval with the volcano-plutonic complexes, covers much of

the western portion of the Iditarod quadrangle (Miller and

Bundtzen, 1987).

Precambrian to late Paleozoic rocks that represent parts of

the Innoko, Ruby, and possibly Kilbuck terranes are exposed in a

narrow belt in the west-central part of the quadrangle.  In the

Iditarod quadrangle, the extension of the Innoko terrane consists

of Mississippian to Jurassic chert and volcanic rock (M.L.

Miller, written commun., 1987).  The Ruby terrane is composed of

greenschist facies metamorphic rocks of probable Precambrian to

Paleozoic age (Angeloni and Miller, 1985).  The possible Kilbuck

terrane equivalent consists of amphibolite grade rocks with a

Proterozoic protolith age, but that have a complex metamorphic

history (Miller and Bundtzen, 1987).  All three units are poorly

exposed as narrow northeast-southwest trending belts.

A relatively minor exposure of ultramafic and mafic rocks

have been mapped in the northern-most central portion of the

quadrangle.  These rocks are probably correlative with the

Jurassic ophiolites of the Yukon-Koyukuk trend further to the

north in the Ophir quadrangle (Miller and Angeloni, 1985).

METHODS OF STUDY

Sample Media

The sampling survey was designed to help relate

geochemical anomalies to specific drainage basins for targeting

mineralized areas.  Stream-sediment samples were collected from

active channels of perennial first-order (unbranched) streams and

second-order (below the junction of two first-order) streams, as

determined from topographic maps (scale 1:63,360).  The area of

the drainage basins ranged from 1 square mile (2.6 square

kilometers) to about 5 square miles (13 square kilometers).

Sampling density was approximately 1 sample site per 9 square

miles (23 square kilometers).  In some cases, swampy areas could

-2-

not be sampled. This is primarily due to the low topographic

relief in much of the quadrangle which results in stagnant

streams with little bed load.

Sample Collection

At individual sample sites, a composite stream-sediment

sample was taken from the active channel and was wet-sieved

through a 10-mesh (2.0 mm) stainless steel screen to remove the

coarse material.  Sediment that passed through the screen was

retained in a 14-inch gold pan until the pan was filled.  A

representative amount of the less than 2.0 mm sediment fraction

was taken directly from the gold pan and saved as the stream

sediment sample.

Sample Preparation

In the laboratory, the stream-sediment samples were air

dried and sieved using an 80-mesh (0.17 mm) stainless steel

sieve.  The portion of the sediment passing through the sieve was

retained and then ground to approximately minus-100-mesh

(0.15 mm).  The processed stream-sediment material was then used

for geochemical analysis.

Sample Analysis

The stream-sediment samples were analyzed for Au using a

flow injection analysis-atomic absorption spectrophotometry

method (FIA-AAS).  The stream sediments were decomposed by

roasting the samples at 700 degrees centigrade in a muffle

furnace prior to digestion with a hydrobromic acid-0.5 percent

bromine solution.  One mL of methyl isobutyl ketone (MIBK) was

used to extract the gold-bromide complex.  Ten gram aliquots of

stream-sediment and Geochemical Exploration Reference (GXR)

standards (Allcott and Lakin, 1975) were used for all analyses.  A

0.1 mL aliquot of the separated MIBK layer was then injected into

an atomic absorption spectrometer using an in-line sample

injection valve.

Stream sediments were analyzed for Hg by a cold vapor

atomic absorption spectrophotometry (AAS) technique described by

Kennedy and Crock (1987).  The samples were first decomposed with

nitric acid and sodium dichromate.  Hydroxylamine hydrochloride/

sodium chloride and stannous chloride were added to the

samples in a continuous flow system.  Mercury vapor was then

measured directly in an optical absorption cell by atomic

absorption spectrophotometry (AAS).  Tellurium and Tl were

determined using the procedure of Hubert and Chao (1985).  Stream

sediments were digested using hydrogen peroxide, hydrofluoric

acid, aqua-regia, and hydrobromic acid-bromine solutions.  The Te

and Tl were concentrated in an MIBK layer and then quantified

using AAS.

The lower limits of determination for Au (FIA-AAS) and for

Hg, Tl, and Te (AAS) are shown in Table 1.  Results for the FIA-

AAS determination of Au in several GXR standards are shown in

Table 2 and are compared to other reported values.

-3-

----------------------------------------------------------------

Table 1.  Lower limits of determination for gold, mercury,

tellurium, and thallium in stream-sediment analysis.

[FIA-AAS = Flow injection analysis-atomic absorption

spectrophotometry; and AAS = atomic absorption

spectrophotometry. Concentrations are in parts per

million.]

Element           Lower determination limit       Method

Gold      (Au)              .005                  FIA-AAS

Mercury   (Hg)              .020                    AAS

Tellurium (Te)              .050                    AAS

Thallium  (Tl)              .050                    AAS

-----------------------------------------------------------------

DATA STORAGE SYSTEM

The geochemical data were entered into the Branch of

Geochemistry's data base.  This data base contains both

descriptive geological information and the analytical data.  Any

or all of this information may be retrieved and converted to a

binary form (STATPAC) for computerized statistical analysis or

publication (VanTrump and Miesch, 1977).  The data in this report

are also available as a hard copy version in Hopkins and others

(1991).

----------------------------------------------------------------

Table 2.  Comparison of gold analyses of GXR standards using

atomic absorption spectroscopy.

[concentrations in ppm; n, number of analyses]

THIS STUDY                 OTHER MEAN VALUES

Sample     Mean         (a)         (b)          (c)         (d)

n=6          n=5         n=5          n=2         n=5

GXR-1  2.94 � .24  3.08 � .07  2.98 � .13  3.03 � .10  3.10 � .02

GXR-2 .022 � .003 .024 � .001 .022 � .003 .034 � .002 .010 �.0003

GXR-4 .433 � .014 .424 � .011 .353 � .021 .462 � .021 .419 � .014

GXR-6 .073 � .003 .076 � .006 .063 � .002 .071 � .001 .086 � .006

-----------------------------------------------------------------

(a) Terashima (1988), (b) Meier (1980), (c) Benedetti and others

(1987), (d) Kontas and others (1986).

-4-

DESCRIPTION OF THE STATPAC FILE

Concentrations for Au, Hg, Tl, and Te in the DATASS.STP

file are given in parts per million (ppm) as indicated.  An S

suffix following the sample numbers designates these samples as

stream sediments.  An "N" indicates that a given element was

looked for, but not detected at the lower limit of determination

shown for that element.  An "L" indicates that the element was

observed, but was below the limit of determination listed.  A "G"

indicates that the concentration for that element was determined

to be greater than the value shown.

Duplicate samples were collected randomly throughout the

study area and are designated with D1, D2, D3, and D4 suffixes in

the DATASS.STP file.  The D2 and D3 suffixes are sample site

duplicates collected from the same stream approximately 100 m

apart.  When enough material was available, the D3 sample was

split in the lab into D3 and D4 samples to estimate analytical

variation within the sample.  The D1 suffixes represent

duplicates collected proximal to the D2 and D3 samples, but on

different streams.  Thus, the D1 samples have a different field

number prefix.  D1 duplicates were not collected with every D2-D3

sample set.  These D1, D2, and D3 samples were collected for

analysis of variance in the study area.

Since 1984, when this project was initiated, numerous

analyses have been performed on the stream-sediment samples from

the Iditarod quadrangle.  As a result, some of the stream

sediments no longer have sufficient material available for

adequate analysis.  For these samples, a ".0B" is listed in the

element column and indicates that this element was not determined

for that sample.  As stated above, only 240 samples were analyzed

for Te and thus, most samples in the data file also have a .0B in

the Te column but are not insufficient samples.  All sample numbers

used in the Iditarod project are provided in the data file for

consistency with the original data set (Gray and others, 1988) and

the sample locality map in Hopkins and others (1991).

-5-

REFERENCES CITED

Allcott, G.H., and Lakin, H.W., 1975, The homogeneity of six

geochemical exploration reference samples, in Elliott,

I.L., and Fletcher, W.K., eds., Geochemical Exploration

1974: Amsterdam, Elsevier, p. 659-581.

Angeloni, L.M., and Miller, M.L., 1985, Greenschist facies

metamorphic rocks of north-central Iditarod quadrangle, in

Bartsch-Winkler, Susan, ed., The U.S. Geological Survey in

Alaska--Accomplishments during 1984: U.S. Geological Survey

Circular 967, p. 19-21.

Benedetti, M.F., DeKersabiec, A.M., and Boulegue, J., 1987,

Determination of gold in twenty geochemical reference

samples by flameless atomic absorption spectrometry:

Geostandards Newsletter, v. 11, p. 127-129.

Bundtzen, T.K., and Laird, G.M., 1983, Geologic map of the

Iditarod D-1 quadrangle, Alaska: Alaska Division of

Geological and Geophysical Surveys, Professional Report 78,

1 plate, scale 1:63,360.

Cady, W.M., Wallace, R.E., Hoare, J.M., and Webber, E.J., 1955,

The central Kuskokwim region, Alaska: U.S. Geological Survey

Professional Paper 268, 132 p.

Gray, J.E., Arbogast, B.F., and Hudson, A.E., 1988, Geochemical

results and sample locality map of the stream-sediment and

nonmagnetic, heavy-mineral-concentrate samples for the

Iditarod quadrangle, Alaska: U.S. Geological Survey Open-

file Report 88-221, 69 p., 1 plate, scale 1:250,000.

Hubert, A.E., and Chao, T.T., 1985, Determination of gold,

indium, tellurium, and thallium in the same sample

digestion of geological materials by atomic-absorption

spectroscopy and two-step solvent extraction: Talanta,

v. 32, p. 383-387.

Hopkins, D.M., Gray, J.E., Hageman, P.L. McDougal, C.M., and

Slaughter, K.E., 1991, Gold, mercury, tellurium, and

thallium data and sample locality map of stream

sediment samples from the Iditarod quadrangle, Alaska:

U.S. Geological Survey Open-file Report 91-283-A, 37 p,

1 plate, scale 1:250,000.

Kennedy, K.R., and Crock, J.G., 1987, Determination of mercury in

geological materials by continuous flow, cold-vapor,

atomic-absorption spectrophotometry: Analytical Letters,

v. 20, p. 899-908.

-6-

Kontas, E., Niskavaara, H., and Virtasalo, J., 1986, Flameless

atomic-absorption determination of gold and palladium in

geological reference samples: Geostandards Newsletter, v.

10, p. 169-171.

Meier, A.L., 1980, Flameless atomic-absorption determination of

gold in geological materials: Journal of Geochemical

Exploration, v. 13, p. 77-85.

Miller, M.L., and Angeloni, L.M., 1985, Ophiolitic rocks of the

Iditarod quadrangle, west-central Alaska (abs): American

Association of Petroleum Geologists Bulletin, v. 69, no. 4,

p. 669-670.

Miller, M.L., and Bundtzen, T.K., 1987, Geology and mineral

resources of the Iditarod quadrangle, west-central Alaska,

in Sachs, J.S., ed., USGS Research on Mineral Resources--

1987, McKelvey Forum Programs and Abstracts, Denver,

Colorado: U.S. Geological Survey Circular 995, p. 46-47.

Terashima, S., 1988, Determination of gold in sixty geochemical

reference samples by flameless atomic absorption

spectrometry: Geostandards Newsletter, v. 12, p. 57-60.

VanTrump, George, Jr., and Miesch, A.T., 1977, The U.S.

Geological Survey RASS-STATPAC system for management and

statistical reduction of geochemical data: Computers and

Geosciences, v. 3, p. 475-488.

-7-