DOCUMENTATION OF MODEL INPUT AND OUTPUT VALUES FOR THE SIMULATION OF

THE GROUND- WATER FLOW SYSTEM AND PROPOSED WITHDRAWALS IN THE NORTHERN

PART OF VEKOL VALLEY, ARIZONA

By  Kenneth J. Hollett

Supplement to Water-Resources Investigations Report 86-4340

_____________________________________________________________________________

U.S. Geological Survey

Open-File Report 94-64

Reston, Virginia

April 1994

-----------------------------------------------------------------------------

DEPARTMENT OF THE INTERIOR

BRUCE BABBITT, Secretary

U.S. GEOLOGICAL SURVEY

Robert M. Hirsch, Acting Director

_____________________________________________________________________________

For additional information write to:

Chief, Office of Ground Water

U.S. Geological Survey

411 National Center

Reston, Virginia  22092

Copies of this report can be purchased from:

U.S. Geological Survey

Earth Science Information Center

Box 25286, MS 517

Denver Federal Center

Denver, Colorado 80225-0046

----------------------------------------------------------------------------

CONTENTS

Page

Abstract			                                        	1

Introduction		     		                             	1

Model grid		     		                         	1

Input and output files			                         	1

References cited		                                    	4

TABLES

Page

Table	1.	File name, Fortran unit number, file size, and

description of input data sets for the Vekol Valley

ground-water flow model			                                3

----------------------------------------------------------------------------

ABSTRACT

This report and the attached 3 1/2-inch diskette contain, in

compressed format, the data sets for the model of ground-water flow in

the Northern part of Vekol Valley.  The data sets can be uncompressed

using a program provided with this report.  The uncompressed files

require approximately 3.1 megabytes of disk space on an IBM-compatible

microcomputer using the MS-DOS operating system, and all files are

presented according to the American Standard Code for Information

Interchange.

INTRODUCTION

A four-layer digital model of the ground-water flow system was

developed as part of a study of the northern part of Vekol Valley,

Arizona, using the MODFLOW program by McDonald and Harbaugh (1988).

Results of the Vekol Valley study are described in a report by Hollett

and Marie (1986).  Although that report summarizes the data input to

the model, detailed documentation of grid location, model input, and

sample output has not been published previously.  This report, which

is a supplement to the report by Hollet and Marie (1986), provides

detailed, electronic documentation of the model data.

MODEL GRID

Organization of the model grid was based on a composite of U.S.

Geological Survey topographical maps, which are identified in Figures

1 and 2 of Hollett and Marie (1986).  For model-grid orientation, the

intersection of the right edge of column 6 and top edge of row 4 is

located about 50 ft. southwest of the intersection of sections 16, 17,

20 and 21 in Township 6 South and Range 1 East (Hollett and Marie,

1986, fig.  8).  The model grid is oriented with rows of model cells

aligned in the east-west direction and columns of model cells aligned

in the north-south direction.

INPUT AND OUTPUT FILES

The original input and output files for the Vekol Valley flow model

were developed on a Prime computer and transferred to an

IBM-compatible microcomputer (operating under MS-DOS version 3.3).

The files were then compressed so that they could be placed on a

single diskette.  The compression program produced a single file that

contains all input and output files.

The diskette included with this report contains a copy of this text

(README.DOC), the library of compressed files (VEKOL.ZIP), and the

decompression program (PKUNZIP.EXE).  The decompression program is

provided by PKWARE, Inc., 7545 North Port Washington Road, Glendale,

Wisconsin.  Permission to copy PKUNZIP is granted freely, provided all

copies acknowledge and reference the authors and distributors of the

program PKUNZIP.exe.  The library can be decompressed by typing

PKUNZIP followed by the library name VEKOL.ZIP and pressing enter.

The decompressed files will be the same as the original Prime computer

files and will consist of text characters represented according to the

American Standard Code for Information Interchange (ASCII).

Files for the Vekol Valley model are on a high-density, double-sided,

soft- sectored, 3 1/2-inch diskette with a capacity of 1.44 megabytes.

The root directory on the diskette contains three files: (1)

README.DOC, (2) the decompression program PKUNZIP.EXE, and (3) the

compressed file VEKOL.ZIP that contains all the input and output files

for the Vekol Valley flow model.  The compressed file VEKOL.ZIP will

unconcompress and form all input and output files.

The MODFLOW ground-water model program runs on a variety of computers,

but procedures for opening input data files are different for

different computers, operating systems and Fortran compilers.  This

report does not address procedures for compiling the MODFLOW program

or opening the model data sets on specific Fortran units.

Two simulations were made using the model: steady-state simulation and

a transient simulation that is a projection of the system response to

a proposed withdrawal plan.  Input files for the steady-state and

transient simulations are listed in table 1.  Record lengths of the

input files are 80 or fewer characters.  There are two output files:

the steady- state simulation output is file VEKSS.OUT, whereas the

transient output is file VEKTRN.OUT.  Each record on the output file

contains 132 or fewer characters.  Data contained in the files are in

units of feet and days.

Table 1. File name, Fortran unit number, file size, and description of

input data sets for the Vekol Valley, Arizona, ground-water flow model

----------------------------------------------------------------------------

File           Fortran      Size         Description of file

name            unit       (Bytes)

----------------------------------------------------------------------------

bas.ss           1          90962        Basic package data for

steady-state simulation

bcf.ss          11          89749        Block-centered flow package data

for steady-state simulation

drn.ss          13           1646        Drain package data for

steady-state simulation

rch.ss          18           3625        Recharge data for

steady-state simulation

sip.ss          19             74        Strongly implicit procedure package

data for steady-state simulation

bas.trn          1          91731        Basic package data for

transient-state simulation;

starting heads are the ending heads

from steady-state simulation

bcf.trn         11         125495        Block-centered flow package

data for transient simulation

wel.trn         12           4210        Well package data for transient

simulatiion

drn.trn         13           1742        Drain package data for transient

simulation

rch.trn         18           3716        Recharge data for transient

simulation

sip.trn         19             74        Strongly implicit procedure package

data for transient simulation

----------------------------------------------------------------------------

REFERENCES CITED

Hollett, K. J. and Marie, J. R., 1986, Simulation of the ground-water

flow system and proposed withdrawals in the northern part of Vekol

Valley, Arizona: U.S. Geological Survey Water- Resources

Investigations Report 86-4340, 68 p.

McDonald, M. G. and Harbaugh, A. W., 1988, A modular three-dimensional

finite- difference ground-water flow model: U.S. Geological Survey

Techniques of Water-Resources Investigations, Book 6, Chapter A1,

586 p.