U.S. DEPARTMENT OF THE INTERIOR / U.S. GEOLOGICAL SURVEY

U.S. GEOLOGICAL SURVEY DIGITAL DATA SERIES DDS-13

International Phase of Ocean Drilling

Seismic Line 1 Demultiplexed Data

Cape Hatteras to Mid-Atlantic Ridge

DISC »18

Demultiplexed Seismic Data

Each file of seismic data is the disc-image of a SEG-Y format tape output during the demultiplexing of the original field data.  The data were written to tape using the 32 bit IBM floating point format (Barry, and others, 1975).  As the data were copied bit-for-bit to the CD-ROM file, this format was preserved.  Each file of sesimic data is named IPD{number}, where {number} is the first shotpoint in the file.  NOTE: FFID indicates the Field File Identification Number assigned each shot as it was recorded.

file:	»IPD13057.SGY

sample rate:	4000.0 microsec

samples per trace:	»1551

data length:	»6204 msec

bytes per trace:	»6444

bytes in file:	»136700172

traces in file:	»21213

first trace:	»876/9 FFID/trace

last trace:	»319/5 FFID/trace

file:	»IPD13279.SGY

sample rate:	4000.0 microsec

samples per trace:	»1551

data length:	»6204 msec

bytes per trace:	»6444

bytes in file:	»137692548

traces in file:	»21367

first trace:	»319/6 FFID/trace

last trace:	»764/12 FFID/trace

file:	»IPD13501.SGY

sample rate:	4000.0 microsec

samples per trace:	»1551

data length:	»6204 msec

bytes per trace:	»6444

bytes in file:	»136732392

traces in file:	»21218

first trace:	»764/13 FFID/trace

last trace:	»206/14 FFID/trace

file:	»IPD13722.SGY

sample rate:	4000.0 microsec

samples per trace:	»1551

data length:	»6204 msec

bytes per trace:	»6444

bytes in file:	»137286576

traces in file:	»21304

first trace:	»206/15 FFID/trace

last trace:	»650/6 FFID/trace

file:	»IPD13944.SGY

sample rate:	4000.0 microsec

samples per trace:	»1551

data length:	»6204 msec

bytes per trace:	»6444

bytes in file:	»103584456

traces in file:	»16074

first trace:	»650/7 FFID/trace

last trace:	»984/48 FFID/trace

Observer Logs

The following files are digitized versions of the original observer's log sheets.  Each file is named OB{number}.{format}, where {number} is the first shot point on the page, and {format} is the graphic format of the file: PCX, TIF, or GIF.  No {format} extension is shown for the file names given below.

OB13050

OB13100

OB13150

OB13200

OB13250

OB13300

OB13350

OB13400

OB13450

OB13500

OB13550

OB13600

OB13650

OB13700

OB13750

OB13800

OB13850

OB13900

OB13950

OB14000

OB14050

OB14100

REFERENCE

Barry, K.M., Cavers, D.A., and Kneale, C.W., 1975, Recommended standards for digital tape formats: Geophysics, v. 40, p. 344-352.