This is a digital copy of a document originally included U.S. Geological Survey Open-File Report 92-349A

============================================================

====================== PLOTSEGY V2.0 =======================

============================================================

PLOTSEGY V2.0

Changes to the Program Version 1.0

PLOTSEGY V2.0 incorporates 4 major changes in function:

1) The ability to change the sample rate used or replace it if not found in the SEG-Y binary header.

2) The ability to scale the display of the data as either "trace normalized" or against a "relative amplitude" either calculated or entered by the user.

3) A trace increment parameter.

4) The ability to annotate traces using one of 3 different trace header values.

Changes 3 and 4 allow PLOTSEGY to display unstacked SEG-Y data and label it appropriately.  Shown in figures 1 - 7 are the user-interface changes seen in PLOTSEGY V2.0.

(1) SAMPLE RATE

After PLOTSEGY has opened the indicated input file and read the binary trace header information, the sample rate found in the binary header is displayed and the user has an opportunity to change it if desired.  The default value found in the binary header is displayed in square brackets (["sample rate", milliseconds]) and may be accepted by pressing <ENTER>.  The user may enter a new value at the prompt, or "quit" or "exit".  The sample rate accepted will be used in all subsequent calculations.

(2A) SCALING - TRACE NORMALIZED

Trace Normalized scaling will compute the average non-zero absolute sample value per trace and use that value to represent 1/2 a trace width when plotting that trace's samples.  Each sample will be displayed relative to that average value for that trace.

(2B) SCALING - RELATIVE AMPLITUDE

Relative Amplitude scaling will take either a value entered by the user or a value calculated from a user-defined set of traces from within the input file and use this value to represent 1/2 a trace width when plotting all samples within the data set.

(3) TRACE INCREMENT

The TRACE INCREMENT parameter allows the user to increment the trace counter through the data.  For unstacked data, this allows the user to display individual traces from each ensemble.

For example, Baikal line 6 SEG-Y demultiplexed data has ensembles composed of 101 traces: 5 auxiliary channels (traces 1 - 5) and 96 data channels (traces 6 - 101).  Ensemble trace 6 is the 1st data trace per ensemble and represents the far data trace. Ensemble trace 101 is the 96th data trace per ensemble and represents the near data trace.  PLOTSEGY counts the physical traces in the file, where the first trace in the file is physical trace number 1.  PLOTSEGY V2.0 will display ALL traces per ensemble, including the auxiliary traces, so they must be included in calculating a starting trace value.  The PLOTSEGY V2.0 parameter 1st_trace_number is the number of the 1st physical trace in the file to display.  If the data set is composed of 10 shot ensembles, the near trace for those 10 shots may be displayed by setting the PLOTSEGY V2.0 parameter 1st_trace_number to file trace 101 and the parameter trace_increment to 101.  This will produce a display using physical traces 101, 202, 303, 404, 505, 606, 707, 808, 909 and 1010, which represent ensemble trace 101 for shots 1 through 10.  A far trace display is possible by setting PLOTSEGY V2.0 parameter 1st_trace_number to physical trace 6 and the parameter trace_increment to 101.  This will produce a display using physical traces 6, 107, 208, 309, 410, 511, 612, 713, 814, and 915, which represent ensemble trace 6 for shots 1 through 10.

For stacked data, this allows the user to simulate the display more data per screen ("panel") then is physically possible.  If the display monitor has VGA resolution, it is

capable of displaying 550 traces per panel.  If the SEG-Y data being displayed has 1100 traces in the data set, the "entire" line may be displayed by setting the PLOTSEGY V2.0 parameter traces_per_panel to 550 and the PLOTSEGY V2.0 parameter trace_increment to 2.  This will decimate the data, displaying every other trace and simulates displaying 1100 traces per panel.

*** NOTE ***

A negative trace increment value will plot the traces in the reverse order to which they are stored in the file.  A trace increment of "-1" and a starting trace number equal to the number of traces in the file will plot the entire data file in reverse order.

(4) TRACE HEADER DISPLAYS

PLOTSEGY V2.0 annotates traces with the physical trace number within the file, labelled "File Trace", and one of 3 trace header values.  The user may select header value CDP, ensemble trace number, or the original field record number (FFID) of that trace.

------------------------------------------------------------

FIGURES

-----------------------------------------------------------

----------------------------------------

SEGY INPUT FILE PARAMETERS

----------------------------------------

CURRENT WORKING DIRECTORY: D:\CPGMS\SEGY\P5

CURRENT FILE PATH: D:\CPGMS\SEGY\P5

enter new path, "exit", "-" to use the current working directory or press ENTER to use current file path: ..\DATA

CURRENT FILE NAME:

enter new name, "exit" or press ENTER to use current: 624-79.SGY

624-79.SGY currently has a sample rate of [   4.000] msec.

Enter a new sample rate if desired.  Press <CR> to accept current value.

sample rate [   4.000]:

-----------------------------------------------------------

FIGURE 1: PLOTSEGY V2.0 SEG-Y Input File Parameters Screen.

-----------------------------------------------------------

-----------------------------

DISPLAY TRACE SCALING METHODS

-----------------------------

NORMALIZED:         PLOTSEGY will average the amplitudes within

the current trace being plotted and uses that

amplitude to represent 1/2 the trace width

for the display of that trace.

RELATIVE AMPLITUDE: The user selects a starting trace and the

number of traces to use.  PLOTSEGY will

average the amplitudes found for all samples

within that range of traces and uses that

amplitude to represent 1/2 a trace width for

the display of all traces in the data set.

Trace scaling: (1) normalized  (2) relative amplitude

scaling [1]: 2

-----------------------------------------------

FIGURE 2: PLOTSEGY V2.0 Scaling Methods Screen.

-----------------------------------------------

Press any key to accept the current relative amplitude to use for

plotting.  Enter a new relative amplitude value or press "C" to

compute a new one.

Current relative amplitude value [   1.000]: C

------------------------------------------------------------

FIGURE 3: User has selected to scale the data using a relative amplitude to be computed ("C").

------------------------------------------------------------

Traces in file 624-79.sgy: [493]

The relative amplitude is computed by averaging the sample values

over a range of traces given by a starting trace number and the

number of traces to use.

starting trace number: 200

number of traces to use: 10

------------------------------------------------------------

FIGURE 4: User has requested the relative amplitude be computed from the average of the sample values found in traces 200 - 209.

------------------------------------------------------------

trace [200] average amplitude [134455664.000]

trace [201] average amplitude [134324672.000]

trace [202] average amplitude [134405600.000]

trace [203] average amplitude [133956936.000]

trace [204] average amplitude [133936984.000]

trace [205] average amplitude [134326336.000]

trace [206] average amplitude [134122296.000]

trace [207] average amplitude [134137880.000]

trace [208] average amplitude [134346800.000]

trace [209] average amplitude [134024192.000]

----------------------------------------------------------

FIGURE 5: PLOTSEGY V2.0 display of calculating the average relative amplitude to be used.

----------------------------------------------------------

----------------------------------------

INPUT FILE CHARACTERISTICS

----------------------------------------

FILE:                  624-79.sgy

sample rate:           4.000 msec

number of samples:     1500

maximum time:          6000 msec

bytes per trace:       6240

bytes in file:         3079920

traces in file:        493

scaling:               RELATIVE AMPLITUDE

scaling factor:        134203736.000

Press any key to continue...

------------------------------------------------------------

FIGURE 6: Input file characteristics displayed by PLOTSEGY V2.0 after computing a new relative amplitude.

------------------------------------------------------------

----------------------------------------

TRACE PLOTTING TYPES

----------------------------------------

TRACE TYPES: (1) wiggle  (2) variable area  (3) both

trace type [3]:

----------------------------------------

PLOT DISPLAY TIMES

----------------------------------------

minimum time [0]:

maximum time [6000]: 1000

timing lines [1000]: 100

<NEW PAGE>

----------------------------------------

TRACE PARAMETERS

----------------------------------------

trace gain [1.000]: 1.5

trace clipping [3]: 1

1st trace number [1]:

Maximum horizontal resolution is 550 traces per panel.

traces per panel [125]:

trace annotation frequency [10]:

trace increment [1]:

Header value to display:

(1) CDP Number   (2) Ensemble Trace Number   (3) Field File

Number

header value [1]:

-------------------------------------------------------

Figure 7: PLOTSEGY V2.0 Trace Information entry screen.

-------------------------------------------------------

============================================================

===================== PLOTSEGY V2.1 ========================

============================================================

PLOTSEGY V2.1

Changes to the Program Version 2.0

(1) PROGRAM STARTUP

PLOTSEGY is started at the DOS prompt, and has 3 optional parameters:

PLOTSEGY {-p DOS path} {-t DOS file descriptor} {-bw}

When started, PLOTSEGY will display a banner page on the user's terminal, which will remain displayed until the user presses any key.    Once the user presses any key, PLOTSEGY will display a File Selection Menu (see FILE SELECTION MENU, below).

Starting PLOTSEGY with no parameters will display to the user a File Selection Menu displaying all the files in the current directory.

Two optional parameters "filter" the files which appear in the File Selection Menu:

OPTION                        DESCRIPTION

-----------                   ------------------------------

-p DOS path                   Allows the user to instruct

PLOTSEGY to look in the

following directory path for

any files.

-t file descriptor            Allows the user to instruct

PLOTSEGY to look for only

those files matching the

following file descriptor.

Starting PLOTSEGY with the "-bw" parameter will display the data in black and white.

EXAMPLES:

---------

PLOTSEGY                           This example starts up

PLOTSEGY, which will look

in the current directory

location for files.

PLOTSEGY -p c:\project1\segy       This example starts up

PLOTSEGY, instructing it

to use only those files

found in the

"C:\PROJECT\SEGY"

subdirectory.

PLOTSEGY -t p*.sgy                 This example starts up

PLOTSEGY, instructing it

to use only those files

in the current directory

location starting with a

"P" and having a ".SGY"

file extension.

PLOTSEGY -t *.sgy -p c:\archive    This example starts up

PLOTSEGY, instructing it

to use all files having a

".SGY" file extension in

the "C:\ARCHIVE"

directory.

(2) FILE SELECTION MENU

The File Selection Menu is a list of files available to

PLOTSEGY as instructed by the user (see PROGRAM STARTUP, above).  The user may navigate through the File Selection Menu as described below:

UP ARROW:  move up 1 line.

DN ARROW:  move down 1 line.

PAGE UP:   move up 1 menu page.

PAGE DOWN: move down 1 menu page.

HOME:      move to 1st menu page.

END:       move to last menu page.

ENTER:     select highlighted file.

ESC:       exit file selection.

============================================================

====================== PLOTSEGY V2.2 =======================

============================================================

PLOTSEGY V2.2

Changes to the Program Version 2.1

PLOTSEGY V2.2 will determine which type of SEG-Y data is present using the SEG-Y defined "data sample format code" read from bytes 3225-3226 of the binary coded file header.  The SEG-Y defined data sample format codes are shown below:

FORMAT CODE                   DATA SAMPLE FORMAT

-----------                   ------------------------------

1                        32 bit IBM floating point

2                        32 bit fixed point

3                        16 bit fixed point

4                        16 bit fixed point with gain

code

============================================================

====================== PLOTSEIS V1.0 =======================

============================================================

PLOTSEIS V1.0

Changes to the PLOTSEGY Version 2.2

Name changed from PLOTSEGY to PLOTSEIS with the additional ability of processing MINI-SOUSIE seismic data files.

(1) PLOTSEIS will interpret any file names ending in .MIN as MINI-SOUSIE seismic data files and process them accordingly.

(2) PROGRAM STARTUP: new command line argument.  PLOTSEIS will display the data with the amplitudes color-coded if started with the command line parameter "-c", as shown below:

PLOTSEIS {-p DOS path} {-t DOS file descriptor} {-c}

The amplitudes are colored according to the following method:

color value = absolute value of the sample amplitude divided by the reference amplitude times 100, i.e.,

colorValue = abs(sampleAmplitude/referenceAmplitude*100)

where the reference amplitude depends on the type of trace scaling specified by the user:

TRACE NORMALIZED:   the reference amplitude is the average positive sample amplitude found for each individual trace.  The colorValue for each sample in an individual trace is determined relative to that value.

RELATIVE AMPLITUDE: the reference amplitude is either entered by the user or the average positive sample amplitude found from a user-specified range of traces.  The colorValue for each sample in an individual trace is determined relative to that value.

Colors are assigned according to the following ranges:

colorValue          Color

0 - 80              blue

80 - 160             light blue

160 - 240             green

240 - 320             light green

320 - 400             red

> 400                   light red

Changing the RELATIVE AMPLITUDE will change how the trace sample amplitudes are colored.  Increasing the RELATIVE AMPLITUDE will shift the colorValue to the lower end of the ranges shown above.  Decreasing the RELATIVE AMPLITUDE will shift the colorValue to the higher end of the ranges.

The TRACE GAIN may be decreased / increased with a corresponding increase / decrease in the RELATIVE AMPLITUDE to maintain the sample horizontal trace excursion.