U.S. DEPARTMENT OF THE INTERIOR / U.S. GEOLOGICAL SURVEY

U.S. GEOLOGICAL SURVEY DIGITAL DATA SERIES DDS-13

International Phase of Ocean Drilling

Seismic Line 1 Demultiplexed Data

Cape Hatteras to Mid-Atlantic Ridge

DISC »07

Demultiplexed Seismic Data

Each file of seismic data is the disc-image of a SEG-Y format tape output during the demultiplexing of the original field data.  The data were written to tape using the 32 bit IBM floating point format (Barry, and others, 1975).  As the data were copied bit-for-bit to the CD-ROM file, this format was preserved.  Each file of sesimic data is named IPD{number}, where {number} is the first shotpoint in the file.  NOTE: FFID indicates the Field File Identification Number assigned each shot as it was recorded.

file:	»IPD2882.SGY

sample rate:	4000.0 microsec

samples per trace:	»1550

data length:	»6200 msec

bytes per trace:	»6440

bytes in file:	»140228160

traces in file:	»21774

first trace:	»165/1 FFID/trace

last trace:	»516/12 FFID/trace

file:	»IPD3057.SGY

sample rate:	4000.0 microsec

samples per trace:	»1550

data length:	»6200 msec

bytes per trace:	»6440

bytes in file:	»78185200

traces in file:	»12140

first trace:	»516/13 FFID/trace

last trace:	»711/62 FFID/trace

file:	»IPD3103.SGY

sample rate:	4000.0 microsec

samples per trace:	»1549

data length:	»6196 msec

bytes per trace:	»6436

bytes in file:	»136942372

traces in file:	»21277

first trace:	»607/1 FFID/trace

last trace:	»950/11 FFID/trace

file:	»IPD3274.SGY

sample rate:	4000.0 microsec

samples per trace:	»1549

data length:	»6196 msec

bytes per trace:	»6436

bytes in file:	»139027636

traces in file:	»21601

first trace:	»950/12 FFID/trace

last trace:	»336/36 FFID/trace

file:	»IPD3407.SGY

sample rate:	4000.0 microsec

samples per trace:	»1549

data length:	»6196 msec

bytes per trace:	»6436

bytes in file:	»137135452

traces in file:	»21307

first trace:	»336/37 FFID/trace

last trace:	»680/15 FFID/trace

Observer Logs

The following files are digitized versions of the original observer's log sheets.  Each file is named OB{number}.{format}, where {number} is the first shot point on the page, and {format} is the graphic format of the file: PCX, TIF, or GIF.  No {format} extension is shown for the file names given below.

OB2850

OB2900

OB2950

OB3000

OB3050

OB3100

OB3150

OB3200

OB3240A

OB3250

OB3290

OB3340

OB3390

OB3431

OB3431G

OB3481

OB3531

REFERENCE

Barry, K.M., Cavers, D.A., and Kneale, C.W., 1975, Recommended standards for digital tape formats: Geophysics, v. 40, p. 344-352.