U.S. GEOLOGICAL SURVEY DIGITAL DATA SERIES DDS-15

CD-ROM Atlas of the Deepwater Parts of the U.S. Exclusive Economic Zone

in the Atlantic Ocean, the Gulf of Mexico, and the Eastern Caribbean Sea

By  D.C. Twichell and C.F. Polloni

Display software by Russell A. Ambroziak

Software documentation by Christine A. Cook

GLORIA sidescan-sonar system

The GLORIA sidescan-sonar system was used on the research vessel Farnella

shown in the following set of pictures with the vehicle mounted on its stern.

Press function key 6 to highlight each figure reference in turn; then press

<ENTER> to display  [gloria],  [farn1],  [farn2],  [farn3].   The following

illustration  includes  a  sequential collage that depicts the vehicle being

loaded on the recovery platform  [glmosaic].

GLORIA creates images (sonographs) that are a record of the acoustic

backscatter from the ocean floor.  These images are created by transmitting a

series of sound pulses and recording their echoes from the sea floor as the

collecting ship moves along a set course.  The sound pulses are transmitted

from two sets of transducers, aimed to port and starboard, which are mounted

in a towed vehicle.  These transducers are tuned so that their beams form a

narrow arc (2.7 m) in the horizontal plane and a broad arc in the vertical

plane. Each transmitted sound pulse thus insonifies a narrow band of sea floor

perpendicular to the ship's track extending from directly beneath the vehicle

to the maximum range on each side.  The sound is reflected from the sea floor

and returned to the transducers on the vehicle, where it is sensed and

converted into electrical signals.  The signals are transmitted along the tow

cable to the ship where they are recorded digitally.  As the ship moves, the

pulses insonify successive bands of sea floor and thus record an acoustic

map.  In the GLORIA system, strong reflections appear as light areas on the

image and weak reflections are dark. This is the reverse of common practice in

sidescan sonar, but makes the records appear more similar to black-and-white

aerial photographs.

A few of the technical features of the GLORIA system are provided here.

For more detailed information and specifications, the reader is referred to

Somers and others (1978).  The towed sidescan vehicle is 8 m long and 0.7 m

in diameter, weighs  2.25 tons in air, and is nearly neutrally buoyant.  The

sonar arrays comprise 120 transducers, arranged in two rows of 30 on each

side of the vehicle.  Each array is 5.3 m long by 40 centimeters (cm) high, a

configuration that gives a horizontal acoustic beam width of 2.7 m and a vert-

ical beam width of 35 m between half-power points; considerable energy actually

radiates outside these limits.  The arrays are designed to confine the energy

as nearly as possible to the plane perpendicular to the track and to fill the

quadrant from directly beneath the towed vehicle up to near horizontal.  The

vehicle is towed about 400 m behind the ship with no active depth control;

at the normal survey speed of 15 km/hour, it is stable at a depth of about

50 m.

Port and starboard transducers operate on slightly different frequencies

in order to minimize crosstalk between the two sides; the port array is

centered at 6.8 kilohertz (kHz) and the starboard array at 6.2 kHz.  The pulse

emitted is a 100-Hz swept band.

The pulse-repetition rate for these surveys was 30 seconds (except for

part of the Puerto Rico EEZ survey that used a 40-second pulse repetition

rate), allowing a maximum swath width of about 45 km under ideal conditions

in water depths greater than 3,000 m.  A longer pulse-repetition rate would

have allowed a greater swath width in deeper water, but would have reduced

data density and, consequently, resolution.  The resolution is determined by

the size of the smallest building blocks of the image, the picture elements,

or pixels.  The size of the pixel is dependent on the number of subdivisions

of an individual scan (1,024), the width of the scan across the track, and

the distance along the track between pulses (determined by the ship's speed

and the pulse-repetition rate).  This effective resolution in the original

data amounts to a pixel size of about 125 m along track and about 50 m in the

across-track direction.  Features on the sea floor that provide targets must

be several hundred meters in size, at least in one direction, in order to be

interpreted.  Of course, a reflection can come from a smaller target than the

nominal resolution size (pixel size).  For example, a scarp that is only a few

meters to a few tens of meters high can provide a reflection and, if several

hundred meters long, can be interpreted.  Furthermore, small-scale surface

features with dimensions of a few centimeters, if spread over a large area,

can collectively affect the echo and their extent can be mapped.

In addition to the GLORIA vehicle, the ship towed a 3.5-kHz sonar system

to record depth and near-surface features [35kHz] and an airgun system that

was the sound source for the single-channel seismic data that were recorded

simultaneously with the sonar data [airgun].

End of text; press the Escape key to continue.