U.S. GEOLOGICAL SURVEY DIGITAL DATA SERIES DDS-15

CD-ROM Atlas of the Deepwater Parts of the U.S. Exclusive Economic Zone

in the Atlantic Ocean, the Gulf of Mexico, and the Eastern Caribbean Sea

By  D.C. Twichell and C.F. Polloni

Display software by Russell A. Ambroziak

Software documentation by Christine A. Cook

SEISMIC DATA

The following text and the accompanying preview screen images

of seismic profiles are taken from USGS Open-File Report 92-289 written

by Hutchinson and others (1992).  These images are included to provide

some information on the subsurface stratigraphy and structures that

underlie the surficial features presented on the GLORIA images for the

Atlantic continental margin.  The complete seismic data set and a more

extensive discussion can be obtained from Hutchinson and others (1992).

Seismic and magnetic data collected during the GLORIA cruises are not in

digital form and thus are not included on this CD-ROM.  These data are

available in the atlases by EEZ-SCAN 85 Scientific Staff (1987a,b) and

EEZ-SCAN 87 Scientific Staff (1991).

In 1978,  the USGS contracted approximately 4,800 km of 48-channel

marine seismic-reflection profiles along the Atlantic continental margin in

support of studies assessing resource potential  and environmental hazards

on the Outer Continental Shelf.  The 21 lines acquired represent state-of

the-art acquisition and processing in 1978 and remain the basis for the

interpretation of much of the structure, stratigraphy, and evolution of the

U.S. Atlantic continental margin (Klitgord and others, 1988).  The

relatively long offsets (3,600-m streamer), long recording time (12 seconds

two-way traveltime), and modest air-gun array (2,000 cubic inches) provide

excellent images of the thick sedimentary basins and occasional images of

subbasement and lower crustal reflections.

Georges Bank Basin: Georges Bank basin contains sedimentary rock that

forms forms a seaward-thickening wedge deposited in one primary and several

ancillary depocenters  created  when North America and Africa rifted apart in

middle Mesozoic time (Schlee and Klitgord, 1988). The  lines  show  images

of  features  such  as the thick wedge of Mesozoic and Cenozoic postrift

deposits, the basement hinge zone, buried  rift  basins,  the postrift

unconformity, the paleoshelf edge, crystalline basement, and oceanic basement,

and occasional intracrustal and Moho  reflections (Grow and others,  1983;

Klitgord and others,  1988; Schlee and Klitgord, 1988).   The dip profiles

(lines  18,   19,  20, 21, 22, and 23) provide excellent images of the

shallow  basement  platform at  the  landward  end  of each profile,  the

deep marginal sedimentary basins beneath the  outer shelf, slope, and rise,

and  the  deep ocean basin at the seaward end  of  each  profile.  Sediment

thickness is in excess of 10 km in the  depocenters and decreases to 0-5 km

along the flanking basement platforms,   such as  the Long Island platform to

the west of Georges Bank and  the Gulf of Maine platform north of Georges Bank.

Two  of  the  lines  presented  here  have  been  the subjects of detailed

studies.   Line  19  across  Georges  Bank lies near two Continental

Offshore Stratigraphic Test wells (Mattick and Libby-French,   1988) and

has been used as a reference profile for  the Georges  Bank basin (Klitgord

and others,   1982;   Poag,   1982; Schlee and Fritsch, 1983;  Schlee and

Klitgord,  1988).   Line 36 crosses  the  Long  Island  platform  and

illustrates   several Triassic-Jurassic  buried  rift  basins (Klitgord and

Hutchinson, 1985; Hutchinson and others, 1986), numerous basement

structures, and an exceptionally well imaged Moho reflection  (Hutchinson

and others,   1986;   Phinney,   1986;   Phinney  and  Roy-Chowdhury,

1989).

Baltimore Canyon Trough:  Lines 24 - 29, 34, 35, and 37 image the

Baltimore  Canyon  trough.  This  basin  is the deepest of the offshore

Mesozoic basins and contains up to 18 km of synrift and postrift deposits

(Grow and others, 1988).  Among the features that are well imaged in the

Baltimore Canyon trough lines are the postrift unconformity, several

synrift half grabens, the basement hinge zone, crystalline basement,

oceanic basement, the paleoshelf edge, and numerous unconformities (Grow,

1980; Schlee and Grow, 1980; Schlee, 1981; Poag, 1985b; Grow and others,

1988).  Some of the best reflection images of Moho from anywhere on the

U.S. East Coast occur on line 37 in  the northern Baltimore Canyon trough

(Hutchinson and others, 1986).  The carbonate complex that forms the Late

Jurassic paleoshelf edge obscures deeper reflections, and so the deep

structure in the center of the basin is poorly imaged on these lines.

Carolina Trough:  Lines 30 - 32 cross the northern part of the

Carolina trough.  This trough is the narrowest of the offshore Mesozoic

basins and contains maximum postrift sediment thickness of about 11-12 km.

Most of the features imaged in the Baltimore Canyon trough are also imaged

in the Carolina trough (Dillon and Paull, 1978; Dillon and others 1979, 1982,

1983).  Distinctive features of the Carolina trough are growth faulting,

abundant salt diapirism, and numerous unconformities in the Tertiary

section related to the strengthening of north-flowing currents (Dillon and

others, 1983; Dillon and Popenoe, 1988).

Lines 25 and 28 from the Baltimore Canyon trough and line 32 from the

Carolina trough have been the subjects of detailed studies.  Line 25

crosses the central Baltimore Canyon trough in the region of densest

drilling (Mattick and Libby-French, 1988) and has been used as a reference

section for the seismic stratigraphy of the trough (Grow, 1980; Grow and

others, 1983; Poag, 1985a).  Line 28 crosses the southern Baltimore Canyon

trough (Grow and others, 1983; Klitgord and others, 1988).  Line 32 has

been intensively studied and modeled in the Carolina trough (Grow and

others, 1983: Hutchinson and others, 1983; Klitgord and others, 1988; Trehu

and others, 1989).

Each seismic line was collected and processed by a contractor.  None of

the data presented here have been migrated.  Most lines have had a prestack

and poststack automatic gain control.  Details of the acquisition geometry and

processing parameters for the other lines are given in the .SPN files in the

directory SEISMIC on the CD-ROM.

End of text; press Escape key to continue.