U.S. DEPARTMENT OF THE INTERIOR

U.S. GEOLOGICAL SURVEY

Analytical results of moss, moss-sediment, and willow

samples from the Iditarod quadrangle, Alaska

By

B.F. Arbogast, B.M. Erickson, J.E. Gray, and J.M. McNeal

Open-file Report 91-380-B Diskette version

This report is preliminary and has not been reviewed for conformity

with U.S. Geological Survey editorial standards and stratigraphic

nomenclature.  Any use of trade names is for descriptive purposes

only and does not imply endorsement by the USGS.

1991

CONTENTS

Page

STUDIES RELATED TO AMRAP .................................... 1

INTRODUCTION ................................................ 1

GEOLOGY ..................................................... 2

METHODS OF STUDY ............................................ 3

Sample Media ........................................... 3

Sample Collection ...................................... 3

Sample Preparation ..................................... 3

Sample Analysis ........................................ 4

DATA STORAGE SYSTEM ......................................... 5

DESCRIPTION OF THE STATPAC FILES ............................ 5

ACKNOWLEDGEMENTS ............................................ 6

REFERENCES CITED ............................................ 6

TABLES

Table 1.  Limits of determination for the spectrographic

analysis of ashed moss and willow samples ......... 9

Table 2.  Limits of determination for the spectrographic

analysis of moss-sediment samples ................ 10

OTHER FILES

README.DOC     An ASCII file describing the diskette version of

this report

TEXTMOSS.DOC   The ASCII file of the text portion of this report

STP2DAT.EXE    A conversion program providing format options for

transforming the .STP files

MOSS.STP       Results of analyses of the ashed moss samples

collected from the Iditarod quadrangle, Alaska

SEDMOSS.STP    Results of analyses of moss-sediment samples

collected from the Iditarod quadrangle, Alaska

WILLOW.STP     Results of analyses of the ashed willow samples

collected from the Iditarod quadrangle, Alaska

-i-

STUDIES RELATED TO AMRAP

The U.S. Geological Survey is required by the Alaskan National

Interests Lands Conservation Act (Public Law 96-487, 1980) to

survey certain Federal lands to determine their mineral values.

Results from the Alaskan Mineral Resource Assessment Program

(AMRAP) must be made available to the public and be submitted to

the President and the Congress.  This report is one of a series of

publications that presents geochemical and mineralogical results

collected from the mineral assessment study of the Iditarod

quadrangle, Alaska.  The data contained in this report are also

available in a hard copy version in Arbogast and others (1991).

INTRODUCTION

In the summer months of 1984-1986, the U.S. Geological Survey

conducted a reconnaissance geochemical survey of the Iditarod

quadrangle, Alaska.  This report lists the analytical results for

samples of moss, moss-sediment, and willow samples that were

collected in addition to the more traditional sampling media for

reconnaissance surveys, such as stream-sediment, heavy-mineral-

concentrate, and stream-water samples.

The Iditarod quadrangle is bounded by 62 to 63 degrees North

latitude and by 156 to 159 degrees West longitude spanning the

Kuskokwim Mountain Range between the Yukon and Kuskokwim Rivers.

The area of the quadrangle is about 6700 square miles (17,300

square kilometers).  The eastern edge of the quadrangle lies

approximately 13 miles (21 kilometers) west of McGrath, the nearest

community having commercial air service.  The quadrangle is

sparsely populated with two small communities at Flat and Takotna

and a few isolated mining camps.  Few roads exist throughout the

quadrangle and access to the area is mostly limited to travel by

air or foot, but boat travel is possible on some of the larger

rivers.

The topography ranges from marshy lowlands of the Innoko

National Wildlife Refuge in the northwest corner, to the more

rugged, glaciated Beaver Mountains in the northeast part of the

quadrangle.  The low point is about 100 feet (30 meters) elevation

and the high point is 4055 feet (1235 meters) in the Beaver

Mountains.  Topographic relief averages about 1200 feet (365

meters) over most of the quadrangle.  The valleys and mountain

slopes are heavily vegetated and timberline is found at an

elevation of about 1000 feet (300 meters).  The approximate height

of the Kuskokwim Mountains averages 1600-1800 feet (490-550 meters)

and consequently, the tops of most ridges and mountains are bare or

tundra covered (McGimsey and others, 1988).

The vegetation in the Iditarod quadrangle is classified as

Taiga or Interior Forest (Kuchler, 1985; Viereck and Dyrness,

1975).  Two main components represent the Interior Forests.  One

component is classified as a "Closed spruce - hardwood forest" and

consists of tall to moderately tall closed forests of white and

-1-

black spruce, paper birch, aspen, alder, and balsam poplar; on

moderate to well-drained sites. The other component, "Open, low

growing spruce forests and treeless bogs" is dominated primarily by

black spruce often interspersed with white spruce, paper birch,

tamarack, and willows, locally interspersed with treeless bogs; on

poorly drained sites usually underlain by permafrost.

GEOLOGY

Cretaceous sedimentary rocks of the Kuskokwim Group (Cady and

others, 1955) form the dominant bedrock in the Iditarod quadrangle.

These rocks consist of thick sequences of intercalated sandstones,

shales, and conglomerates (Bundtzen and Laird, 1983).  Rocks of the

Kuskokwim Group primarily represent deep water turbidite facies,

but small amounts of shallow shoreline facies rocks are also found

in the sequences (Miller and Bundtzen, 1987).  These rocks have

been deformed into northeast trending synclines and anticlines;

high-angle faults appear to parallel these folds.  A major

northeast trending strike-slip fault, the Iditarod-Nixon Fork

fault, transects the central portion of the quadrangle.

Late Cretaceous to early Tertiary volcanic-plutonic complexes

intrude or overlie the Kuskokwim sedimentary rocks at several

localities.  These complexes consist of basalt and andesite

volcanic flows that are in fault contact with or overlie monzonite

plutons.  An extensive felsic to mafic volcanic field, that is

coeval with the volcanic-plutonic complexes, covers much of the

western portion of the Iditarod quadrangle (Miller and Bundtzen,

1987).

Precambrian to late Paleozoic rocks that represent parts of

the Innoko, Ruby, and possibly Kilbuck terranes are exposed in a

narrow belt in the west-central part of the quadrangle.  In the

Iditarod quadrangle, the extension of the Innoko terrane consists

of Mississippian to Jurassic chert and volcanic rock (M.L. Miller,

written commun., 1987).  The Ruby terrane is composed of

greenschist facies metamorphic rocks of probable Precambrian to

Paleozoic age (Angeloni and Miller, 1985).  The possible Kilbuck

terrane equivalent consists of amphibolite grade rocks that yield

a Proterozoic protolith age, but that have a complex metamorphic

history (Miller and Bundtzen, 1987).  All three units are poorly

exposed as narrow northeast-southwest trending belts.

A relatively minor exposure of ultramafic and mafic rocks have

been mapped in the northern-most central portion of the quadrangle.

These rocks are probably correlative with the Jurassic ophiolites

of the Yukon-Koyukuk trend further to the north in the Ophir

quadrangle (Miller and Angeloni, 1985).

-2-

METHODS OF STUDY

Sample Media

Aquatic bryophytes (mosses) have been recognized as a suitable

sampling media for mineral prospecting (Shacklette, 1984).  Several

unique features which make them an ideal sampling media are:  1)

mosses are long lived, which minimizes seasonal fluctuations in

their chemistry caused by seasonal flow rates in streams, 2)

species identification is not necessary because the differences in

the chemistry between moss species does not present the problem as

it does in higher plants, and 3) mosses do not have to be dried and

processed immediately to prevent the growth of mold or decay which

can alter vegetation chemistry (Shacklette, 1984).  Samples of

moss-sediment, stream sediment trapped within mosses, have also

been used successfully as a geochemical exploration medium

(Hedderly-Smith and Glavinovich, 1991).  Sediment trapped within

moss has a great ion-exchange capacity, and has been shown to

concentrate rare earths and other metals (Smith, 1976).

Sample Collection

Samples of moss, and sediment trapped within the mosses, were

collected from many of the same localities as the stream-sediment

(Gray and others, 1988a) and stream-water samples (Gray and others,

1988b), as part of the overall geochemical survey of the area.

Moss samples could not be located at 288 of the 1151 sampling sites

within the quadrangle.  Locations in latitude and longitude for the

samples are provided in the .STP files.  A map plate showing site

localities for all geochemical samples collected during this

project is also included in Arbogast and others (1991).

The moss samples consisted of living vegetation material

collected within the active stream channel, the sides of streams,

or from over banks.  The streams sampled were primarily first-order

(unbranched) and second-order (below the junction of two

first-order) streams as shown on USGS topographic maps (scale

1:63,360).  The stream sediment trapped within the mosses was later

washed from the moss samples in the lab and saved as the moss-

sediment samples.

At 35 sites, willow samples were collected where mosses

could not be located.  The willow samples were a composite of twigs

and leaves.  The terminal 6-10 inches of several willows were

collected within 10 to 15 feet of each other at each site, in the

active stream channel when possible.  The moss and willow sample

results should not be combined for purposes of interpretation.

Sample Preparation

The preparation method used on the moss samples collected the

first year (1984) was very different from the procedure that

developed by the third year (1986).  Caution should be exercised in

interpreting the analytical results due to this change in

techniques.  Results from the other more traditional sampling media

should be included in any interpretation.  For purposes of

-3-

comparison and interpretation, sample site localities I0001-I0409

are from 1984, I0410-I0999 are from 1985, and I1000-I1571 are from

1986.  The 1984 moss samples were placed directly from their sample

bag into a 1400-mL beaker containing tap water, alien objects

removed, and the moss sample squeezed by hand.  Water in the beaker

was changed five times or more until the water remained somewhat

clean.  All rinse water was decanted into a funnel with a 1500 to

2000-mL filter and the moss-sediment saved for analysis.  The moss

material was then placed into a clean Hubco bag for drying. Only

about 7-10 samples a day could be washed using this procedure.

A procedure developed by Lenarcic and Pirc (1986) was adapted

for preparation of the samples collected in 1985.  The dry moss

samples were pounded with a rubber mallet while still in their

sample bag to aid in the removal of the moss-sediment and to cut

the amount of time required for preparation.  After pounding, the

sample bag was transferred to a flour sifter held over a beaker

allowing the moss material and sediment to separate.  The remaining

moss material was then bagged in a plastic bag, filled with tap

water and agitated.  The water was drained and replaced a minimum

of four times. Distilled water was used on the final rinse.

Preparation time advanced to approximately seven samples an hour.

The procedure used for the samples collected in 1986 was

modified again to speed preparation time.  Instead of rinsing the

pounded, sifted moss material in the small zip-loc bags, they were

put directly into a 1400 mL beaker half-full of water, agitated,

and squeezed by hand.  Four rinses followed by a distilled water

rinse were still necessary, but the amount of preparation time was

reduced considerably.  Seven samples could now be washed in a

half-hour.

All cleaned moss and willow samples were dried at 50 degrees

centigrade in a forced air oven, ground in a Wiley Mill, and ashed

at 500 degrees centigrade.  Analytical results are presented on

ash-weight basis.  Percent ash content of these samples is not

available.  The moss-sediment samples were dried under forced air

at ambient temperature and sieved using an 80-mesh (0.17 mm)

stainless steel sieve.  The portion of the sediment that passed

through the sieve was saved.  This minus-80-mesh sediment was then

ground to approximately minus-100-mesh (0.15 mm) and used for

chemical analysis.

Sample Analysis

The moss, moss-sediment, and willow samples were analyzed for

34 elements using a semiquantitative, direct-current arc emission

spectrographic (SQS) method (Grimes and Marranzino, 1968).  The

elements analyzed and their lower limits of determination are

listed in tables 1 and 2.  The ashed moss and willow, and the moss-

sediment, SQS results were obtained by visual comparison of spectra

derived from the sample against spectra obtained from plant

standards and standards made from pure oxides and carbonates,

respectively.  Standard concentrations are geometrically spaced

-4-

over any given order of magnitude of concentration as follows: 100,

50, 20, 10, and so forth.  Samples whose concentrations are

estimated to fall between those values are assigned values of 70,

30, 15, etc.  The precision of the analytical method is

approximately plus or minus one reporting interval at the 83

percent confidence level and plus or minus two reporting intervals

at the 96 percent confidence level (Motooka and Grimes, 1976).

Values determined for the major elements (iron, magnesium, sodium,

calcium, and titanium) are given in weight percent; all

others are given in parts per million (micrograms/gram).  The two

different SQS lower limit of determinations found for lanthanum,

tungsten, and cobalt (table 2) are due to a change in standard

operating procedure that occurred during this project.  The

elements gallium, germanium, and sodium were added to the SQS

method during the course of the project and therefore, most samples

were not analyzed for these elements.

In addition to the SQS analysis, the moss, moss-sediments, and

willow samples were analyzed for uranium using a UV-fluorescence

method following sample digestion with nitric acid (Centanni and

others, 1956).  The normal working range for the uranium analysis

of a 0.5 g sample was 0.05 ppm (lower limit of determination) and

100 ppm (upper limit of determination).   However, some samples did

not have enough material for the uranium analysis.  As a result,

when less than 0.5 g of material was analyzed, the upper or lower

limits of determination were adjusted to estimate uranium

concentrations in these samples.  Thus, the upper and lower limits

of determination for uranium vary accordingly in these instances.

DATA STORAGE SYSTEM

Upon completion of all analytical work, the analytical results

were entered into the Branch of Geochemistry's computer data base.

This data base contains both descriptive geological information and

analytical data.  Any or all of this information may be retrieved

and converted to a binary form (STATPAC) for computerized

statistical analysis or publication (VanTrump and Miesch, 1977).

The data in this report are also available as a hard copy version

in Arbogast and others (1991).

DESCRIPTION OF STATPAC FILES

The STATPAC files MOSS.STP, SEDMOSS.STP, WILLOW.STP list the

results of analyses for the samples of moss, moss-sediment, and

willow, respectively.  An executable data conversion program,

STP2DAT.EXE has been included in this report to provide various

format options into which the .STP files may be changed (Grundy and

Miesch, 1987).  In the STATPAC files, column element headings that

include "S" represent emission spectrographic analyses and "INST"

-5-

indicates UV-fluorescence analyses.  A letter "N" following

geochemical data indicates that a given element was looked for, but

not detected at the lower limit of determination shown for that

element.  If an element was observed but was below the lowest

reporting value, an "L" was entered following of the lower limit of

determination.  If an element was observed, but was above the

highest reporting value, a "G" was entered following of the upper

limit of determination.  If an element was not looked for in a

sample, a "0B" was entered in place of an analytical value.  An "H"

is listed in a few instances were interferences hindered the

ability to determine sample concentrations for an element.

Duplicate samples were collected randomly throughout the study

area and are designated with D1, D2, D3, and D4 suffixes in the

STATPAC files.  The D2 and D3 suffixes are sample site duplicates

collected from the same stream approximately 500 ft (150 m) apart.

When enough material was available, the D3 sample was split in the

lab into D3 and D4 samples to estimate analytical variation within

the sample.  The D1 suffixes represent duplicates collected

proximal to the D2 and D3 samples, but on different streams.  Thus,

the D1 samples have a different field number prefix.  D1 duplicates

were not collected with every D2-D3 sample set.  The duplicate

samples were collected for analysis of variance in the study area.

ACKNOWLEDGMENTS

The authors wish to thank Peter Folger, Karen Kelley, Jerry

Gaccetta, and Robert Carlson for assisting with sample collection.

Greg Thurston and Tom Peacock are acknowledged for their assistance

in preparation of the moss samples.  Ted Roemer performed the

uranium analyses and John Bullock assisted with the SQS analyses

for this study.

REFERENCES CITED

Arbogast, B.F., Erickson, B.M., Gray, J.E., and McNeal, J.M., 1991,

Analytical results and sample locality map of moss, moss-

sediment, and willow samples from the Iditarod quadrangle,

Alaska: U.S. Geological Survey Open-file Report 91-380-A, 101

p., 1 plate, scale 1:250,000.

Angeloni, L.M., and Miller, M.L., 1985, Greenschist facies

metamorphic rocks of north-central Iditarod quadrangle, in

Bartsch-Winkler, Susan, ed., The U.S. Geological Survey in

Alaska-Accomplishments during 1984:  U.S. Geological Survey

Circular 967, p. 19-21.

-6-

Bundtzen, T.K., and Laird, G.M., 1983, Geologic map of the Iditarod

D-1 quadrangle, Alaska:  Alaska Division of Geological and

Geophysical Surveys, Professional Report 78, 1 map, scale

1:63,360.

Cady, W.M., Wallace, R.E., Hoare, J.M., and Webber, E.J., 1955, The

central Kuskokwim region, Alaska: U.S. Geological Survey

Professional Paper 268, 132 p.

Centanni, F.A., Ross, A.M., and DeSesa, M.A., 1956, Fluorometric

determination of uranium:  Analytical Chemistry, v. 28, p.

1651.

Gray, J.E., Arbogast, B.F., and Hudson, A.E., 1988a, Geochemical

results and sample locality map of the stream sediment and

nonmagnetic, heavy-mineral-concentrate samples from the

Iditarod quadrangle, Alaska:  U.S. Geological Survey Open-File

Report 88-221, 69 p., 1 plate, scale 1:250,000.

Gray, J.E., Ryder, J.L., Sanzolone, R.F., McHugh, J.B., and

Ficklin, W.H., 1988b,  Analytical data and sample locality map

for stream water samples from the Iditarod quadrangle, Alaska,

U.S. Geological Survey Open-File Report 88-55, 23 p., 1 plate,

scale 1:250,000.

Grimes, D.J., and Marranzino, A.P., 1968, Direct-current arc and

alternating-current spark emission spectrographic field

methods for the semiquantitative analysis of geologic

materials:  U.S. Geological Survey Circular 591, 6 p.

Grundy, W.R., and Miesch, A.T., 1987, Brief descriptions of STATPAC

and related statistical programs for the IBM Personal

Computer: U.S. Geological Survey Open-file Report 87-411-A, 34

p.

Hedderly-Smith, D.A., and Glavinovich, P.S., 1991, Moss-mat stream

sediment sampling in the Threemile Creek-Black Lake Area,

southeastern Alaska, in Explore, Lavin, O.P., ed., The

Association of Exploration Geochemists Newsletter: Denver,

Colorado, no. 71, p. 14-17.

Kuchler, A.W., 1985, Map of the potential natural vegetation of

Alaska, University of Kansas (revised 1985), in National Atlas

of the United States of America: Department of Interior,

U.S. Geological Survey.

Lenarcic, T., and Pirc, S., 1986, Rapid method of cleaning aquatic

moss:  Short note in Journal of Geochemical Exploration, v.

27, p. 213-216.

-7-

McGimsey, R.G., Miller, M.L., and Arbogast, B.F., 1988, Paper

version of analytical results, and sample locality map for

rock samples from the Iditarod quadrangle, Alaska:  U.S.

Geological Survey Open-File Report 88-421-A, 110 p., 1 plate

scale 1:250,000.

Miller, M.L., and Angeloni, L.M., 1985, Ophiolitic rocks of the

Iditarod quadrangle, west-central Alaska (abs):  American

Association of Petroleum Geologists Bulletin, v. 69, no. 4, p.

669-670.

Miller, M.L., and Bundtzen, T.K., 1987, Geology and mineral

resources of the Iditarod quadrangle, west-central Alaska, in

Sachs, J.S., ed., USGS Research on Mineral Resources, 1987,

Programs and Abstracts, Denver, Colorado: U.S. Geological

Survey Circular 995, p. 46-47.

Motooka, J.M., and Grimes, D.J., 1976, Analytical precision of

one-sixth order semiquantitative spectrographic analyses: U.S.

Geological Survey Circular 738, 25 p.

Shacklette, H.T., 1984, The use of aquatic bryophytes in

prospecting:  Journal Geochemical Exploration, v. 21, p.

89-93.

Smith, D.C., 1976, Moss-trapped stream material as a prospecting

medium:  Journal Geochemical Exploration, v. 5, p. 338-341.

VanTrump, George, Jr., and Miesch, A.T., 1977, The U.S. Geological

Survey RASS-STATPAC system for management and statistical

reduction of geochemical data:  Computers and Geosciences, v.

3, p. 475-488.

Viereck, L.A.and Dyrness, C.T., 1975, A preliminary classification

system for vegetation of Alaska:  U.S. Department of

Agriculture, Forest Service, Pacific Northwest Forest and

Range Experiment Station, General Technical Report PNW-106, 38

p.

-8-

TABLE 1.  Limits of determination for the spectrographic analysis

of ashed moss and willow samples, based on a 5-mg

sample

_______________________________________________________________

Element        Lower limit of      Upper limit of

determination       determination

________________________________________________________________

Percent

________________________________________________________________

Iron (Fe)          0.005                 5

Magnesium (Mg)      .01                 10

Sodium (Na)         .005                 5

Titanium (Ti)       .001                 1

________________________________________________________________

Parts per million

________________________________________________________________

Silver (Ag)            0.1              500

Arsenic (As)         200              5,000

Gold (Au)              2                500

Boron (B)              5              1,000

Barium (Ba)           20             20,000

Beryllium (Be)         0.5              100

Bismuth (Bi)           1                500

Cadmium (Cd)           1                500

Cobalt (Co)            5              1,000

Chromium (Cr)          5              1,000

Copper (Cu)            1              5,000

Gallium (Ga)           2                100

Germanium (Ge)         2                100

Lanthanum (La)        20                500

Manganese (Mn)        10             10,000

Molybdenum (Mo)        5                500

Niobium (Nb)          20                500

Nickel (Ni)            5              1,000

Lead (Pb)             10              5,000

Antimony (Sb)         50              5,000

Tin (Sn)               5                500

Strontium (Sr)       100              5,000

Vanadium (V)           5              1,000

Tungsten (W)          50              1,000

Yttrium (Y)           10                500

Zinc (Zn)            100             20,000

Zirconium (Zr)        10              1,000

Indium (In)            2                100

Lithium (Li)         200             10,000

Thallium (Tl)          2                100

-9-

TABLE 2.  Limits of determination for the spectrographic analysis

of moss-sediment samples, based on a 10-mg sample.

Element        Lower limit of      Upper limit of

determination       determination

________________________________________________________________

Percent

________________________________________________________________

Calcium (Ca)       0.05                 20

Iron (Fe)           .05                 20

Magnesium (Mg)      .02                 10

Sodium (Na)         .2                   5

Titanium (Ti)       .002                 1

________________________________________________________________

Parts per million

________________________________________________________________

Silver (Ag)         0.5                 5,000

Arsenic (As)      200                  10,000

Gold (Au)          10                     500

Boron (B)          10                   2,000

Barium (Ba)        20                   5,000

Beryllium (Be)      1                   1,000

Bismuth (Bi)       10                   1,000

Cadmium (Cd)       20                     500

Cobalt (Co)         5 or 10             2,000

Chromium (Cr)      10                   5,000

Copper (Cu)         5                  20,000

Gallium (Ga)        5                     500

Germanium (Ge)     10                     100

Lanthanum (La)     20 or 50             1,000

Manganese (Mn)     10                   5,000

Molybdenum (Mo)     5                   2,000

Niobium (Nb)       20                   2,000

Nickel (Ni)         5                   5,000

Lead (Pb)          10                  20,000

Antimony (Sb)     100                  10,000

Scandium (Sc)       5                     100

Tin (Sn)           10                   1,000

Strontium (Sr)    100                   5,000

Thorium (Th)      100                   2,000

Vanadium (V)       10                  10,000

Tungsten (W)       20 or 50            10,000

Yttrium (Y)        10                   2,000

Zinc (Zn)         200                  10,000

Zirconium (Zr)     10                   1,000

-10-