oooooooooooooooooooooooooooooooooooooooooooooooooooooooooooooo

DEPARTMENT OF THE INTERIOR

U.S. GEOLOGICAL SURVEY

FASPU English and Metric Version:

Analytic Petroleum Resource Appraisal

Microcomputer Programs for Play Analysis

Using a Reservoir-Engineering Model

By

Robert A. Crovelli and Richard H. Balay

This report is preliminary and has not been reviewed for

conformity with U.S. Geological Survey editorial standards. Any

use of trade, product or firm names is for descriptive purposes

only and does not imply endorsement by the U.S. Government.

Although this program has been used by the U.S. Geological

Survey, no warranty, expressed or implied, is made by the USGS as

to the accuracy and functioning of the program and related

program material, nor shall the fact of distribution constitute

any such warranty, and no responsibility is assumed by the USGS

in connection therewith.

1990

oooooooooooooooooooooooooooooooooooooooooooooooooooooooooooooooo

1.  INTRODUCTION

FASPU is a prototype package of programs for IBM PC/XT/AT (and

compatible) microcomputers to assess the resource potential of

undiscovered oil and gas resources using a play analysis method.

Play analysis is a general term for various geologic models and

probabilistic methods of analyzing a geologic play for petroleum

potential. FASPU means Fast Appraisal System for Petroleum -

Universal version.

This is an extensive modification of another program (FASP) which

the USGS used in the assessment of the petroleum resource

potential in the Arctic National Wildlife Refuge of Alaska

(Crovelli, 1988).

The FASPU package is a revision of earlier releases of the same

information (see Crovelli and Balay, 1987, and Crovelli and

Balay, 1989). The earlier programs FASPUE and FASPUM have been

combined within one interface to allow the user to choose either

English or metric (SIU) units. The underlying methodology in both

systems of units is unchanged from that used in FASPUE and

FASPUM. This new version is compatible with the computer data

files created by the earlier releases.

The geologic model used in this study is a universal type of

reservoir engineering model. It is a generalization of the site-

specific geologic model developed by the U.S. Department of the

Interior and applied by the USGS in petroleum assessments of the

National Petroleum Reserve in Alaska and the Arctic National

Wildlife Refuge (U.S. Department of the Interior, 1979; White,

1979). The probabilistic methodology used in those two

assessments was a Monte Carlo simulation method.

FASPU is based upon an analytic method using probability theory,

rather than a Monte Carlo simulation method. Conditional

probability theory is applied, along with many laws of

expectation and variance. The risk structure considers both the

uncertainty of the presence of the assessed petroleum resource

and its amount if present. The geostochastic system can be

applied in explored as well as frontier areas.

Separate programs exist for assessing individual plays and for

aggregating a set of plays. The programs produce resource

estimates of crude oil, nonassociated gas, dissolved gas, and

total gas for a geologic play and an aggregation in terms of

probability distributions.

The reader should refer to Crovelli and Balay (1988) for a

description of the reservoir engineering model and the analytic

probabilistic methodology. Other useful references are Crovelli

(1985, 1987a, 1987b) and Crovelli and Balay (1986).

All of the geologic play data required by this model are entered

on an oil and gas appraisal data form, and later transcribed to

computer data files for processing by the FASPU software.

An IBM-PC compatible 5.25" diskette containing the executable

programs and documentation files for FASPU is distributed in

association with this Open-File Report. The files on the diskette

are:

FASPU.COM      The main system program;

UEED.CHN       The data entry editor module for English units;

UMED.CHN       The data entry editor module for metric units;

UERE.CHN       The resource assessment module for English

units;

UMRE.CHN       The resource assessment module for metric

units;

FASPAG.COM     The aggregation program;

FASPU.DOC      The documentation file;

INSTAL2D.BAT   The automatic installation script for a 2-

diskette computer (5.25" diskette);

INSTALHD.BAT   The automatic installation script for a hard

disk computer;

TESTUE.DAT     A file of hypothetical sample data for test-

running the FASPU system using English units;

TESTUM.DAT     A file of hypothetical sample data for test-

running the FASPU system using metric units.

2.  COMPUTER REQUIREMENTS

The computer requirements for running FASPU are the following:

IBM PC/XT/AT or compatible computer

Monochrome or color monitor

8087 Math Coprocessor

2 diskette drives (5.25"), or 1 diskette and a hard disk

256 K memory

Printer able to print 132 characters on a line

The program does not require a graphics adapter. There may be

compatibility problems in running FASPU on some display hardware,

or with some older versions of the DOS operating system. The

development system was MS-DOS 3.1.

3.  FASPU INSTALLATION GUIDE

FASPU consists of several modules, performing three main

functions:

FUNCTION                 MODULE(S) INVOLVED

---------------------------    -----------------------------

Data entry and editing          FASPU.COM

UEED.CHN     UMED.CHN

Resource assessment             FASPU.COM

UERE.CHN     UMRE.CHN

Aggregation of assessment       FASPAG.COM

The first two functions are combined under the supervision of the

shell program FASPU.COM. When FASPU.COM runs, it requests a data

file name and allows the user to pass this file into either

UEED.CHN (or UMED.CHN) for data entry and editing, or into

UERE.CHN (or UMRE.CHN) for processing of the data in the file.

The user needs only run FASPU.COM and enter information requested

by the program; loading and execution of the other modules is

done by FASPU.COM.

This section covers the installation of the necessary executable

files to generate a FASPU working system on either a dual 5.25"

diskette computer or a computer with a diskette and a hard disk.

Information on running the FASPU system after it is installed

appears in section 4.

3.1  Installation for a Diskette System

---------------------------------------

To install FASPU on a PC with two 5.25" diskette drives:

*   Format a new diskette, using the standard form of the FORMAT

command to include the DOS operating system on the new

diskette. For help in doing this, refer to the MS-DOS or PC-

DOS manual for your machine. Label this diskette "FASPU SYSTEM

DISK."

*   Insert the FASPU issue diskette which was shipped to you into

another diskette drive. Log onto this disk by typing the DOS

command

disk:

In place of "disk" substitute the letter of the diskette drive

containing the issue disk, and be sure to include the colon

after the drive letter.

*   Run the FASPU installation program for two diskettes by typing

INSTAL2D  disk:

This time in place of "disk" substitute the drive letter of

the disk on which you are installing FASPU. This command takes

care of creating the needed directory, named \FASPU, on the

new system diskette and copying the necessary files into it.

An example of the install command is INSTAL2D  B:

*   Make a backup copy of the system diskette, and keep it in a

secure place for recovery of the system in case the working

system disk is damaged and becomes unusable. Refer to the MS-

DOS or PC-DOS manual for your machine for help in copying a

diskette.

*   Format another diskette to use as a FASPU data disk. As FASPU

runs, it generates additional files on the same disk with the

data. It is not advisable to put the data files on the same

disk with the system, as the system disk has little free

space.

When running FASPU, always have the system disk as the default

disk. You cannot run FASPU if you are logged onto a disk

different from the one where FASPU resides.

3.2  Installation for a Hard Disk System

----------------------------------------

To install FASPU on a PC with a hard disk and one diskette drive,

use the following procedures. NOTE: the FASPU installation

program creates a new directory on the hard disk, named \FASPU.

In the unlikely case there is  already such a directory on the

hard disk, the installation may fail. You should eliminate the

existing \FASPU directory (refer to your MS-DOS or PC-DOS manual)

before running the installation procedure.

*   Insert the FASPU issue diskette which was shipped to you into

a diskette drive, and log onto this disk by typing the DOS

command

disk:

In place of "disk" substitute the letter of the diskette drive

containing the issue disk.

*   Run the FASPU installation program for hard disk systems by

typing

INSTALHD   disk:

In place of "disk" substitute the drive letter of the hard

disk where you want to install FASPU, and be sure to include

the colon after the drive letter. An example of the install

command is INSTALHD  C:

*   Use an ASCII text editor (such as EDLIN, Sidekick, or a word

processor) to modify the PATH command in the AUTOEXEC.BAT file

in the root directory of the startup disk. At the end of the

PATH command, add the text

;disk:\FASPU

where "disk" is the same as in the preceding paragraph. For

example, if you are installing to disk C, append to the PATH

command the phrase

;C:\FASPU

If the AUTOEXEC.BAT file doesn't have a PATH command in it,

add a line at the end of the file which says

PATH disk:\FASPU

An example of this is PATH C:\FASPU.

If the startup disk doesn't have an AUTOEXEC.BAT file, use an

ASCII text editor to create one, and put the above PATH

command in it.

*   To make the changes in AUTOEXEC.BAT take effect before running

FASPU for the first time, reboot the system by keying CTRL-

ALT-DEL. This makes it possible to use FASPU immediately. From

this point on, the modifications in AUTOEXEC.BAT will take

effect automatically every time the computer is turned on.

It is possible to put the data files in the same directory with

the system, but you may prefer to package the data files in a

different directory. You can also keep the data on a diskette

instead of on the hard disk, but this is less convenient.

When running FASPU, always have the hard disk containing FASPU as

the default disk; it will not run from any other default disk.

However you can change to any desired directory on the hard disk

for the data files, and run FASPU from there. FASPU generates a

number of new files while processing the data, and puts them in

the same directory and on the same disk as the data file.

3.3  Disabling One Set of Units

-------------------------------

If all your work will be restricted to one set of units (English

or metric), you can save storage space and simplify the operation

of the system by deleting (or renaming) the FASPU system files

related to the units you will not be using. The following table

shows which files to eliminate.

TO DISABLE ...           ... DELETE THESE FILES

-------------------      ---------------------------

English units            UEED.CHN      UERE.CHN

metric units             UMED.CHN      UMRE.CHN

FASPU detects which system files are present and adjusts its

units accordingly.

4.  FASPU USER'S GUIDE

Before running FASPU, it must be correctly installed on your PC.

If this has not been done, you must run through the installation

procedure described in section 3. After installation is complete,

proceed with the operation of the system, described in this

section.

In most respects, operation of FASPU is the same whether you are

using a dual diskette system or a hard disk system. These two

installations have slightly different methods of starting out;

both cases are covered separately below.

4.1  Starting FASPU - Diskette Computers (two diskette drives)

--------------------------------------------------------------

*   Put the FASPU system disk into a diskette drive and log onto

this drive.

*   Put a formatted data disk into another drive.

*   After the DOS prompt, type

FASPU

*   When FASPU asks for a file name, type a name of the form

drive:datafile.DAT

In place of "drive" substitute the drive letter of the disk

where you plan to keep data files, and in place of "datafile"

substitute your chosen data file name. For example, you might

give the file name B:FILE5.DAT. The data file name must be 8

or fewer letters or digits, not counting the .DAT suffix. The

.DAT is not required as part of the name, but it is

recommended. You can use another suffix if preferred, but you

must avoid the suffixes .REL, .AGG, and .AGL as FASPU uses

these suffixes for its own output files. FASPU doesn't check

the legality of the file name, so if you enter an illegal one,

the program aborts with a cryptic "I/O Error F1."

4.2  Starting FASPU - Hard Disk Computers

-----------------------------------------

*   Log onto the hard disk in your computer which contains FASPU.

Change to the directory where you plan to keep data files.

*   After the DOS prompt, type:

FASPU

*   When FASPU asks for a file name, type a name of the form

datafile.DAT

In place of "datafile" substitute the name you have chosen for

your data. For example, you might give the file name

SOUTH.DAT. The data file name must be 8 or fewer letters or

digits. The suffix .DAT is not required, but it is

recommended. You can use another suffix if preferred, but you

must avoid the suffixes .REL, .AGG, and .AGL as FASPU uses

these suffixes for its own output files.

The file name may include a directory path prefix if needed,

for example  \MYDATA\TESTRUN.DAT. FASPU doesn't check the

legality of the file name, so if you enter an illegal one, the

program aborts with an "I/O Error F1."

4.3  Operation of FASPU - Both Diskette and Hard Disk Computers

---------------------------------------------------------------

After loading FASPU and specifying a data file name, both

diskette and hard disk systems work the same way. A diskette

system works a bit slower.

If the named file does not already exist, FASPU asks if you want

to create a new file with that name. Answer Y or N. The N

response is provided as an escape in case you really wanted an

existing file but typed its name incorrectly.

FASPU now asks you to specify which units you want for your data.

Type letter E for English or M for metric. (This question is

omitted if you have disabled one of the sets of units as

described in paragraph 3.3).

The first data entry screen appears. It includes a bar at the top

of the screen with the name of the file being edited, and a bar

at the bottom showing a menu of the control keys: the arrow keys,

the RETURN and TAB keys, PgUp and PgDn. The details of working

with the data input editor are described in the next section.

If the named file is an old one, then FASPU gives the options of

editing the file or sending the file directly to the FASPU

assessment program. The details of this are given later.

4.4  Operation of the Data Entry Editor - New Files

---------------------------------------------------

FASPU has a screen-oriented data entry module. The program

accepts keyboard input of parameters through a series of 9

display screens. Each screen contains cells for entry of a group

of parameters. You can step sequentially through the cells on a

screen by striking the RETURN key after entering each value. The

right arrow key and the TAB key have the same function as RETURN.

By using the cursor keypad and other control keys you can browse

randomly through the cells on the visible screen, and through the

adjacent screens, until you come to the cell where you want to

enter or edit data. These are the screen controls:

Up arrow key     This key jumps the cursor to the next line

above the current line. But if the cursor is

already on the first line of the screen, there

is no effect.

Down arrow key   This key jumps the cursor to the next line

below the current line. But if the cursor is

already on the last line of the screen, there

is no effect.

Right arrow key  This key jumps the cursor to the next cell to

the right of the current cell. If the cursor is

already on the last cell on the current line,

the cursor jumps to the first cell on the next

line below. But if the cursor is already on the

last cell of the screen, there is no effect.

Left arrow key   This key jumps the cursor to the next cell to

the left of the current cell. But if the cursor

is already on the leftmost cell of the current

line, there is no effect.

RETURN key       This has the same effect as the right arrow

key.

TAB key          This also has the same effect as the right

arrow key.

PgUp key         This key jumps to the previous screen. But if

the cursor is already on screen 1, FASPU

displays "No previous screen."

PgDn key         This key jumps to the next screen. But if the

cursor is already on the last data screen

(screen 9), FASPU displays "No following

screen."

ESC key          This key can be pressed from any cell on any

screen to escape from the data entry function.

FASPU then gives the option to send the data to

the resource assessment module, to return for

more editing on the data, or to quit.

For a new file, FASPU has already entered dummy data into some of

the cells. You can step through the cells and substitute actual

values for your application.

The size of each cell limits the amount of space for that entry.

If a cell is completely filled with characters, the editor jumps

to the next cell.

As you enter values, FASPU monitors the entries for correctness.

If an entry has an error in it, FASPU displays an error message

on the screen and waits for you to retype correctly. However if

you realize there is a typing error in the current cell, you can

backspace over the bad characters and then retype.

NUMERIC ENTRIES. If the parameter is a number, it can be entered

in any reasonable form: real numbers can be in fixed point

notation (like 3.1416) or in floating point scientific notation

(like 2.386E+3, which means 2.386 times 10 to the power 3).

Wherever FASPU requests real numbers, integers are also

acceptable. When FASPU requests integers (as in the number of

geologic prospects), these must be typed without a decimal point

(like 34, never as 34.0).

PROBABILITY ENTRIES. If the entry being requested is a

probability, it must be entered as a real number in the range 0.0

to 1.0. A probability which is out of range causes an error

message.

PERCENT ENTRIES. If the entry being requested is a percent, it

must be entered as a real number in the range 0 to 100. If it is

out of range, an error message is given.

FRACTILE ENTRIES. The lists of fractiles on screen #3 must be in

nondecreasing order. If they are not, an error message is given.

The fractiles for Depth on this screen must be strictly

increasing; if any two of the fractiles are equal, an error

results.

GEOLOGIC VARIABLE SCREENS. The data on screens 4 through 8

specify the parameters for the five geologic variables in the

FASPU assessment: reservoir pressure (Pe), reservoir temperature

(T), gas/oil ratio (Rs), oil formation volume factor (Bo), and

gas compressibility factor (Z). Each of these variables can be

specified as one of four function types:  (a) zoned linear: a set

of from 1 to 4 linear functions of depth, with transition depths

to separate the zones; (b) exponential function of depth; (c)

power function of depth; and (d) logarithmic function of depth.

For each of the geologic variables, one of the function types is

highlighted; and this is the function type currently selected.

Use the left/right cursor keys to highlight one of the other

function types, and the mathematical form of the function changes

on the screen to show the currently selected formula.

If the selected function type is zoned linear, a line appears on

the display to show the number of zones (1 to 4), and one of

these is highlighted to show the current selection. Use the

left/right cursor keys to highlight one of the other numbers, and

the mathematical form of the function changes on the screen to

show the currently selected formula and number of zones. If 3 or

more zones are specified, the entries in the cells for transition

depths must be strictly increasing, else an error results.

When you have entered all the data on the nine screens, press the

ESC key. FASPU prints the DO WHAT menu:

Do what with this file?

E  Return to editing

S  Save file; Send to the FASPU assessment program

X  Exit and save file

Q  Quit without saving.

Press the letter key corresponding to the action wanted. Either

upper or lower case letters are accepted. If you type X, FASPU

saves the data in the file directory and returns to DOS; only the

new version of the file is kept, and it replaces the old version,

if any. If you type Q, the system terminates without saving the

newly created (or edited) file; this option is used if you

realize the current edits are useless and you don't want to save

this version of the data. If you type S, FASPU saves the file in

the directory, then sends the data directly to the FASPU

assessment module. If you type E, then FASPU re-enters the data

entry module for further review and editing. This is the same as

editing an old file, and this process is described next.

4.5  Operation of the Data Entry Editor - Old Files

---------------------------------------------------

If you specify the name of an already existing data file when

first entering FASPU, the system shows a DO WHAT menu similar to

the one in the paragraph above.

If the file needs to be edited before sending it on to the FASPU

assessment program, press the E key. Then FASPU runs through the

data entry screens as before, but it now shows the cells already

filled with the parameters in the existing file. If the existing

values on a screen are correct, press PgUp or PgDn to review the

other screens. If the content of a cell needs to be changed, move

the cursor to that cell and type the new entry to replace the

old. When the edited file is correct, press ESC while any data

input screen is visible to return to the DO WHAT menu.

5.  COMPUTING THE RESOURCE ASSESSMENT

Whenever the DO WHAT menu appears on the screen, you have the

option to send the current data file to the FASPU assessment

module. To do this, press the S key. The FASPU assessor reads the

input data file, performs the assessment of resources, and

produces two output files: a listing which summarizes the input

data and displays an estimate of resources in the play; and a

file of parameters which can be fed into the aggregator program

to combine estimates of several plays after they are processed by

FASPU.

The summary listing file is named by FASPU to agree with the data

file name, but having the file name suffix .REL. The aggregation

data file is named by FASPU to agree with the data file name, but

with the suffix .AGG. For example, if the original data file name

is TEST.DAT, then FASPU generates new files with names TEST.REL

and TEST.AGG.

When the assessment program starts, it asks if you want to

include computations of fractiles for the number of resource type

accumulations in the play. These computations are potentially the

most time consuming part of the FASPU method. Including them is

optional because their effect on running time depends on a

complex interaction between the number of prospects fractiles and

several other parameters. They probably should be included for a

first run (type y or Y in response to the query) because for most

data sets the computing time is acceptably low. If the program is

running too long, you must abort FASPU by entering CTRL-C, then

restart it on the same data file. This time, enter n or N when

asked if you want to include the number-of-accumulations

computations, and the program should finish in a short time. In

this case, FASPU prints a row of dashes on the summary output

file for number of accumulations to show that this computation

was bypassed.

Then, FASPU asks for a run number. It must be given as an

integer. This is intended to help keep track of the results after

several different runs of the same play with variations in the

data input on each run. The run number is printed at the top of

the .REL output listing.

Now FASPU runs its assessment. It may take from 10 seconds to

several minutes to complete, depending on the content of the data

file and on whether you have elected the option to compute number

of accumulations. When FASPU is finished, it returns to DOS, and

it leaves the .REL and .AGG output files, as well as the source

data file, in the file directory.

Numeric errors may occur during the assessment: things like

overflow or division by zero. FASPU attempts to trap such errors,

but their occurrence is due to a complex interaction between the

many input parameters, and the program may not catch them all. If

you see an message like Run-time Error 01, PC = 802A, then FASPU

will terminate. Restart the program and use it to examine the

input data file for data entries which are missing, inconsistent

with the other values, or out of reasonable range.

6.  PRINTING THE OUTPUT

The printer used to print the file resulting from a FASPU

assessment run should be able to print 132 columns across a page.

Scroll the paper in the printer until the print head is

positioned about 3 lines below the tear-off perforations.

Now enter the standard DOS command:

PRINT  playdata.REL

For "playdata" substitute whatever name you gave to the data file

when you created it through the FASPU editor. Examples of this

command are PRINT TEST3.REL and PRINT B:WESTERN.REL. The output

file is printed on two pages.

The other file output by FASPU, the one with the .AGG suffix, is

also printable, but it is intended to be read by another program,

not by people.

7.  RUNNING THE FASPAG AGGREGATION MODULE

FASPAG is a program which combines the resource assessments from

previous runs of FASPU on two or more plays. FASPAG aggregates

the plays and produces two new output files: (1) a summary

listing file showing input values and output estimates; and (2)

another .AGG file which can later participate in a higher level

aggregation.

To run FASPAG, type the command

FASPAG

FASPAG requests a title line for this aggregation. This is the

title which is printed across the top of the summary output file,

and is for documentation purposes. Type any desired title line.

Now FASPAG asks for the name of a file on which to write the

output listing. This must be a legal DOS file name consisting of

a first name of 8 or fewer letters or digits and a suffix .AGL.

An example of an output file name is ALLUSA.AGL. If you use a

different suffix, FASPAG changes it to .AGL. If you want the

output listing file to go onto a disk different from the default

disk, include a disk drive or directory prefix with the file

name, as in the examples B:ALLUSA.AGL or \PROJECTX\ALLUSA.AGL.

FASPAG does not check the legality of the file name, so if you

specify an illegal one, the program aborts with an I/O Error F1.

FASPAG uses the output file name to generate the aggregation file

name: this has the same first name as the .AGL file, but the

suffix is .AGG.

Then FASPAG asks you to enter a degree of dependency between the

plays to be aggregated. This must be a real number between 0.0

and 1.0 inclusive. If the entry is out of bounds, FASPAG waits

for you to re-enter it.

Finally, FASPAG goes into a mode which allows you to type any

number of aggregation file names to join in the aggregation.

These must be files with type suffixes of .AGG which were

generated by earlier runs of FASPU or FASPAG. After typing the

names of all the aggregation files which are to participate in

the aggregation, just press RETURN to exit. FASPAG completes the

assessment, saves the output files and returns to DOS.

There is a possibility that FASPAG will encounter computational

problems with certain aggregation data files having unusually

small unconditional play probabilities. This problem is

unpredictable, as the computation is a complex interaction

between these probabilities and several other parameters of the

aggregation. If this happens, FASPAG outputs a message like

arithmetic fault in nonassociated gas, and it enters rows of

dashes on the output summary file to indicate the bypassed

fractile computations. However, a valid aggregation output file

is still produced, and this .AGG file can participate in further

aggregations.

To print FASPAG'S summary output file, use the DOS command

PRINT  aggdata.AGL

For "aggdata" substitute the name given above for the aggregation

output listing file.