DEPARTMENT OF THE INTERIOR

U.S. GEOLOGICAL SURVEY

Diskette version of the analytical results of

rock samples from the eastern

Goodnews Bay quadrangle, southwest Alaska

By

J.E. Gray, B.M. Adrian, P.L. Hageman, and J.E. Kilburn

Open-File Report 92-8-B Diskette version

This report is preliminary and has not been reviewed for conformity

with U.S. Geological Survey editorial standards and stratigraphic

nomenclature. Any use of trade names is for descriptive purposes

only and does not imply endorsement by the USGS.

U.S. Geological Survey, DFC, Box 25046, MS 973, Denver, CO, 80225

1992

CONTENTS

Page

STUDIES RELATED TO AMRAP............................................ 1

INTRODUCTION.......................................................  1

GENERAL GEOLOGY AND MINERAL OCCURRENCES............................  1

METHODS OF STUDY...................................................  2

Sample Collection.............................................  2

Sample Analysis...............................................  2

DATA STORAGE SYSTEM................................................  3

DESCRIPTION OF THE STATPAC FILE....................................  3

ACKNOWLEDGMENTS....................................................  4

REFERENCES CITED...................................................  4

TABLES

Table 1.  Limits of determination for spectrographic analysis of

rock samples.............................................  7

Table 2.  Other analytical methods used and limits of

determination............................................  8

FILES

README.DOC     An ASCII file describing the diskette version of

this report

TEXTROCK.DOC   The ASCII file of the text portion of this report

ROCK.STP       Results of analyses of rock samples collected from

the eastern Goodnews Bay quadrangle, Alaska

STP2DAT.EXE    A conversion program providing format options for

transforming the ROCK.STP file

STUDIES RELATED TO AMRAP

The U.S. Geological Survey is required by the Alaska National

Interests Lands Conservation Act (Public Law 96-487, 1980) to survey

certain Federal lands to determine their mineral potential.  Results from

the Alaska Mineral Resource Assessment Program (AMRAP) must be made

available to the public and submitted to the President and Congress.  This

report is one of a series of publications that presents geochemical results

collected during the mineral assessment study of the Goodnews Bay

quadrangle, Alaska.  Geochemical data for rock samples collected from

selected areas within the eastern portion of the Goodnews Bay quadrangle

are presented here.  The data in this report are also available in a hard

copy version in Gray and others (1992).

INTRODUCTION

Between 1975 and 1977, the U.S. Geological Survey conducted a

reconnaissance geochemical survey of the Goodnews Bay, Hagemeister Island,

and Nushagak Bay quadrangles as part of the mineral resource assessment of

the region.  Sample media collected included stream sediments, nonmagnetic

heavy-mineral concentrates derived from stream sediments, and organic

materials.  A description of the sampling methods and analytical procedures

used during the reconnaissance survey, along with a tabulation of the

geochemical data and a site locality map for the samples are given in

Cieutat and others (1988).

Using the geochemical data of Cieutat and others (1988), Kilburn and

Jones (1992) and Jones and Kilburn (1992) delineated a number of

geochemically anomalous areas considered favorable for the presence of

metallic mineral resources.  Geochemical anomalies identified in the

eastern part of the Goodnews Bay quadrangle were the subject of a follow-up

field investigation in the summer of 1990.  During this follow-up study,

altered and mineralized rocks were collected in most areas identified by

the geochemical reconnaissance study as anomalous.  In addition, rock

samples were collected from two poorly studied mineral occurrences, a

sphalerite rich vein along the Togiak River (Hoare and Cobb, 1977), and a

Hg-rich occurrence near the Ongivinuck River (Coonrad and others, 1978).

This report presents only the locations and the analytical data for rock

samples collected during the 1990 follow-up study.  The data listed in this

report were interpreted by Kilburn and others (1992).  The mineral assessment

report of the Goodnews Bay study appears in Kilburn and others (in press).

GENERAL GEOLOGY AND MINERAL OCCURRENCES

The eastern Goodnews Bay quadrangle is underlain primarily by

northeast-striking rocks of the Togiak tectonostratigraphic terrane (Jones

and others, 1987; Box, 1985).  This structurally complex terrane is

characterized by a thick sequence of Jurassic graywacke and Jurassic and

Early Cretaceous interbedded breccia, tuff, basalt, tuffaceous siltstone,

and chert.  The volcanic, volcaniclastic, and sedimentary rocks of the

Togiak terrane are intruded locally by felsic stocks of Late Cretaceous to

-1-

early Tertiary age.  Major structural features, some of which appear to

delineate terrane boundaries, trend northeast to north-northeast (Hoare and

Coonrad, 1978).  The most detailed account of the geology and a correlation

of rock units is given in Hoare and Coonrad (1978).

The Bristol Bay mining region, which is regarded as one mining

district, encompasses the eastern part of the Goodnews Bay quadrangle

(Cobb, 1973).  Only a few small Hg-, Cu-, and Zn-bearing quartz veins and

reports of scattered Au placers were documented in the eastern Goodnews Bay

region (Eberlein and others, 1977) prior to the 1990 follow-up study.

Quartz veins include a sphalerite-bearing occurrence with minor amounts of

chalcopyrite in altered pillow lava just south of the Kashaiak Mountains

along the Togiak River (Berg and Cobb, 1967), the cinnabar-stibnite-

realgar-orpiment-bearing Kagati Lake prospect hosted in a Late Cretaceous

granitic stock located several km east of Kagati Lake near Mount Oratia

(Sainsbury and MacKevett, 1965), and small copper-rich veins hosted in a

Late Cretaceous to Tertiary granitic stock in the Pistuk Peak-Togiak Lake

region (Eakins, 1968; Hoare and Cobb, 1977).  Gold-bearing placers were

reported at several sites along Trail and Rainy Creeks, and minor placer

gold was reported in creeks that drain the ridge southwest of Sunshine

Valley and streams entering Elva Lake (Hoare and Cobb, 1977).  Newly

discovered base- and precious-metal-bearing veins in the eastern Goodnews

Bay quadrangle are described in Kilburn and others (1992).

METHODS OF STUDY

Sample Collection

Geochemical sampling for this follow-up study consisted of rock

samples collected from outcrop, ridge float, or talus slopes.  Outcrop rock

samples were collected as composite chip samples that were as

representative as possible of the exposed rocks.  Most samples were

collected from altered or mineralized rocks.  This report contains

analytical data for approximately 100 rock samples that were collected from

areas in the eastern portion of the Goodnews Bay quadrangle.

Sample Analysis

Rock samples were crushed and pulverized to minus-100-mesh (150 microns)

using a disk mill with ceramic plates.  The pulverized samples were

analyzed for a variety of elements by different chemical methods.  Samples

were analyzed for 35 elements using a semiquantitative, direct-current arc

emission spectrographic method (Grimes and Marranzino, 1968).

Spectrographic results were determined by visually comparing spectra

derived from the sample against spectra obtained from laboratory reference

standards.  Standard concentrations are geometrically spaced over any given

order of magnitude of concentration such that values reported for each

sample are reported in a geometric sequence 10, 15, 20, 30, 50, 70, 100,

etc.  The elements determined by the spectrographic method and their limits

of determination are listed in table 1.

-2-

The samples were also analyzed for 10 elements by inductively coupled

plasma-atomic emission spectrometry after a partial digestion of a 1-gram

aliquot and organic solvent extraction (Motooka, 1988).  Table 2 lists the

elements analyzed by inductively coupled plasma-atomic emission

spectrometry and limits of determination.  In addition, gold was determined

by graphite furnace atomic absorption spectrophotometry following a

hydrobromic acid-bromine digestion of a 10-gram aliquot and an organic

solvent extraction (O'Leary and Meier, 1986).  Tungsten was determined by a

visible spectrophotometric method by decomposing the sample with nitric,

hydrofluoric, and hydrochloric acids (Welsch, 1983).  Mercury was

determined by a modified version of the cold-vapor atomic absorption

spectrophotometry method of Kennedy and Crock (1987).  Lower limits of

determination for Au, W, and Hg by these methods are also listed in table 2.

Discrepancies in analyses for certain elements duplicated by different

analytical methods, such as values determined for Au, may be attributable

to the particulate nature of minerals that contain Au, different sample

aliquots used, and different extraction procedures.  The atomic absorption

spectrophotometry analysis of Au provides the most statistically

representative results due to the larger sample aliquot analyzed.  For

example, a 10-gram sample aliquot is used for the atomic absorption

analysis, whereas a 10-milligram sample aliquot is used in the

spectrographic technique.

DATA STORAGE SYSTEM

Upon completion of the analytical work, the results were entered into

a computer-based file as part of the USGS Rock Analysis Storage System

(RASS) database.  This database contains both descriptive geological

information and analytical data.  Any of this information may be retrieved

and converted to a binary form (STATPAC) for computerized analysis or

publication (VanTrump and Miesch, 1976).  The data in this report are also

available in a hard copy version in Gray and others (1992).

DESCRIPTION OF STATPAC FILE

The STATPAC file ROCK.STP contains only the sample locations in latitude

and longitude and the analytical data.  For sample descriptions refer to Gray

and others (1992).  An executable data conversion program STP2DAT.EXE (Grundy

and Miesch, 1987) is also contained on the diskette that provides various

format options into which the ROCK.STP file may be changed.  The analytical

method for each element shown in the ROCK.STP file is abbreviated as a suffix

in the column headings with the designations "S", "P", "A", and "V"

indicating semiquantitative emission spectrography, inductively coupled

plasma-atomic emission spectrometry, atomic absorption spectrophotometry, and

visible spectrophotometry analyses, respectively.

The letter "N" in the data file indicates that an element was looked for

but not observed at the concentration shown, while an "L" indicates that an

element was observed but present in concentrations below the lower limit of

determination shown.  A "G" was entered in the data file after the upper

limit of determination if an element was observed but was present in

-3-

concentrations above this value.  Lower and upper limits of determination

for the inductively coupled plasma-atomic emission spectrometry method

listed in the data file may be variable due to variable sample aliquot

weight, dilution of an analytical aliquot, or instrumental interference

correction.  Values determined for the major elements, Fe, Mg, Ca, Na, Ti,

and P are given in weight percent; all other values are in parts per million

(micrograms/gram).

ACKNOWLEDGMENTS

We would like to acknowledge R. Goldfarb and S. Box for their

assistance with sample collection in the field.  In addition, we thank D.

Abrams, J. Kelley, and C. Motooka for sample preparation; J. Motooka and B.

Roushey for chemical analyses; and K. Slaughter for assistance with report

preparation and data reduction.

-4-

REFERENCES CITED

Berg, H.C., and Cobb, E.H., 1967, Metalliferous lode deposits of Alaska:

U.S. Geological Survey Bulletin 1246, 254 p.

Box, S.E., 1985, Terrane analysis of the northern Bristol Bay region,

southwestern Alaska, in Bartsch-Winkler, Susan, ed.: U.S. Geological

Survey Circular 967, p. 32-37.

Cieutat, B.A., Goldfarb, R.J., and Speckman, W.S., 1988, Analytical results

and sample locality map of stream sediment, heavy-mineral concentrate,

and organic material samples from the Goodnews, Hagemeister Island,

and Nushagak Bay quadrangles, southwest Alaska: U.S. Geological Survey

Open-File Report 88-591, 187 p.

Cobb, E.H., 1973, Placer deposits of Alaska: U.S. Geological Survey

Bulletin 1374, 213 p.

Coonrad, W.L., Hoare, J.M., Taufen, P.M., and Hessin, T.D., 1978,

Geochemical analysis of rock samples in the Goodnews and Hagemeister

Island quadrangles region, southwestern Alaska: U.S. Geological Survey

Open-File Report 78-9-H, scale 1:250,000.

Eakins, G.R., 1968, A geochemical investigation of the Wood River-Tikchik

Lakes area, southwestern Alaska: Alaska Division of Mines and Minerals

Geochemical Report 17, 31 p.

Eberlein, G.D., Chapman, R.M., Foster, H.L., and Gassaway, J.S., 1977,

Table describing known metalliferous and selected nonmetalliferous

mineral deposits in central Alaska: U.S. Geological Survey Open-File

Map 77-168-D.

Grimes, D.J., and Marranzino, A.P., 1968, Direct-current arc and

alternating current spark emission spectrographic field methods for

the semiquantitative analysis of geologic materials: U.S. Geological

Survey Circular 591, 6 p.

Gray, J.E., Adrian, B.M., Hageman, P.L., and Kilburn, J.E., 1992,

Analytical results and sample locality maps of rock samples from the

eastern Goodnews Bay quadrangle, southwest Alaska: U.S. Geological

Survey Open-File Report 92-8-A, 21 p.

Grundy, W.R., and Miesch, A.T., 1987, Brief descriptions of STATPAC

and related statistical programs for the IBM Personal

Computer: U.S. Geological Survey Open-file Report 87-411-A, 34

p.

Hoare, J.M., and Cobb, E.H., 1977, Mineral occurrences (other than mineral

fuels and construction materials) in the Bethel, Goodnews, and Russian

Mission quadrangles, Alaska: U.S. Geological Survey Open-File Report

77-156, 98 p.

-5-

Hoare, J.M., and Coonrad, W.L., 1978, Geologic map of the Goodnews Bay and

Hagemeister Island quadrangles region, southwestern Alaska: U.S.

Geological Survey Open-File Report 78-9-B, scale 1:250,000.

Jones, D.L., Silberling, N.L., Coney, P.J., and Plafker, George, 1987,

Lithotectonic terrane map of Alaska (west of the 141st meridian): U.S.

Geological Survey Miscellaneous Field Studies Map MF-1874-A, scale

1:250,000.

Jones, J.L., and Kilburn, J.E., 1992, Geochemical map showing the

distribution of selected elements in heavy-mineral concentrate samples

from the Goodnews Bay, Hagemeister Island, and Nushagak Bay

quadrangles, Alaska: U.S. Geological Survey Miscellaneous Field

Studies Map MF-2186, 2 sheets, scale 1:250,000.

Kennedy, K.R., and Crock, J.G., 1987, Determination of mercury in geologic

materials by continuous flow, cold-vapor, atomic-absorption

spectrophotometry: Analytical Letters, v. 20, p. 899-908.

Kilburn, J.E., Box, S.E., Goldfarb, R.J., and Gray, J.E., 1992,

Geochemically anomalous areas in the eastern Goodnews Bay 1 X 3 degree

quadrangle, southwest Alaska, in Bradley, D.C., and Ford, A.B., eds.,

Geologic Studies in Alaska by the U.S. Geological Survey, 1990: U.S.

Geological Survey Bulletin 1999, p. 156-162.

Kilburn, J.E., Goldfarb, R.J., Griscom, Andrew, and Box, S.E., in press,

Map showing metallic mineral resource potential in the Goodnews Bay,

Hagemeister Island, and Nushagak Bay 1 X 3 degree quadrangles, Southwest

Alaska: U.S. Geological Survey Miscellaneous Field Studies Map.

Kilburn, J.E., and Jones, J.L., 1992, Geochemical map showing the

distribution of selected elements in stream sediments from the

Goodnews Bay, Hagemeister Island, and Nushagak Bay quadrangles,

Alaska: U.S. Geological Survey Miscellaneous Field Studies Map MF-

2355, 2 sheets, scale 1:250,000.

Motooka, J.M., 1988, An exploration geochemical technique for the

determination of preconcentrated organometallic halides by ICP-AES:

Applied Spectroscopy, v. 42, no. 7, p. 1293-1296.

O'Leary, R.M., and Meier, A.L., 1986, Analytical methods used in

geochemical exploration, 1984: U.S. Geological Survey Circular 948,

48 p.

Sainsbury, C.L., and MacKevett, E.M., Jr., 1965, Quicksilver deposits of

southwestern Alaska: U.S. Geological Survey Bulletin 1187, 89 p.

VanTrump, George, Jr., and Miesch, A.T., 1976, The U.S. Geological Survey

RASS-STATPAC system for management and statistical reduction of

geochemical data: Computers and Geosciences, v. 3, p. 475-388.

-6-

Welsch, E.P., 1983, A rapid geochemical spectrophotometric determination of

tungsten with dithiol: Talanta, v. 30, p. 876-878.

___________________________________________________________________________

Table 1.  Limits of determination for the spectrographic analysis of rock

samples, based on a 10-mg sample.

Elements        Lower determination limit     Upper determination limit

Percent

Iron (Fe)                0.05                          20

Magnesium (Mg)           0.02                          10

Calcium (Ca)             0.05                          20

Sodium (Na)              0.2                            5

Titanium (Ti)            0.002                          1

Phosphorous (P)          0.2                           10

Parts per million

Silver (Ag)              0.5                        5,000

Arsenic (As)           200                         10,000

Gold (Au)               10                            500

Boron (B)               10                          2,000

Barium (Ba)             20                          5,000

Beryllium (Be)           1                          1,000

Bismuth (Bi)            10                          1,000

Cadmium (Cd)            20                            500

Cobalt (Co)             10                          2,000

Chromium (Cr)           10                          5,000

Copper (Cu)              5                         20,000

Gallium (Ga)             5                            100

Germanium (Ge)          10                            100

Lanthanum (La)          50                          1,000

Manganese (Mn)          10                          5,000

Molybdenum (Mo)          5                          2,000

Niobium (Nb)            20                          2,000

Nickel (Ni)              5                          5,000

Lead (Pb)               10                         20,000

Antimony (Sb)          100                         10,000

Scandium (Sc)            5                            100

Tin (Sn)                10                          1,000

Strontium (Sr)         100                          5,000

Thorium (Th)           100                          2,000

Vanadium (V)            10                         10,000

Tungsten (W)            20                         10,000

Yttrium (Y)             10                          2,000

Zinc (Zn)              200                         10,000

Zirconium (Zr)          10                          1,000

-7-

Table 2.  Other analytical methods used and limits of determination*.

[ICP-AES, Inductively Coupled Plasma-Atomic Emission

Spectrometry; CVAAS, Cold-Vapor Atomic Absorption

Spectrophotometry; GFAAS, Graphite Furnace Atomic Absorption

Spectrophotometry; VS, Visible Spectrophotometry].

Analytical         Lower limit         Upper limit

Element             Method              (ppm)

Silver (Ag)         ICP-AES             0.045               1,500

Arsenic (As)        ICP-AES             0.6                 3,000

Gold (Au)           ICP-AES             0.15                2,400

Bismuth (Bi)        ICP-AES             0.6                 1,500

Cadmium (Cd)        ICP-AES             0.03                  500

Copper (Cu)         ICP-AES             0.03                1,200

Molybdenum (Mo)     ICP-AES             0.09                1,500

Lead (Pb)           ICP-AES             0.6                12,000

Antimony (Sb)       ICP-AES             0.6                   800

Zinc (Zn)           ICP-AES             0.03                  500

Gold (Au)           GFAAS               0.002

Mercury (Hg)        CVAAS               0.02

Tungsten (W)        VS                  1

* NOTE:  Lower and upper limits of determination for the ICP-AES

method listed in this table are nominal, and in the ROCK.STP file may

be variable.  The variability in limits of determination for an

element is due to variable sample aliquot weight, dilution of an

analytical aliquot, or instrumental interference correction.

-8-