DIGITAL ARCHIVE DESCRIPTION OF GRID DATA

CLIENT                       U.S.G.S

NAME OF SURVEY               AEROMAGNETIC SURVEY of WESTERN NEVADA

AREA NAME                    EXCELSOIR MOUNTAINS, SOUTHERN WASSUK RANGE and EXCELSIOR ADD-ON

CONTRACTOR                   FUGRO AIRBORNE SURVEYS

JOB CODE                     655

GRID ARCHIVE NAME            NEVADA_TF.GRD

FORMAT                       GEOSOFT BINARY GRID

CONTENT                      TOTAL MAGNETIC INTENSITY

CREATION DATE                February 15, 2001

NUMBER of ROWS and COLUMNS   640 1024

PIVOTAL POSITION (X,Y)       331087 4212084

CELL SIZE                    83m

UNIT                         nT

UNIT FACTOR                  0.001

MINIMUM VALUE (nT)           -555.284

MAXIMUM VALUE (nT)            846.571

MEAN VALUE (nT)               -71.181

GRID ARCHIVE NAME            NEVADA_RADAR.GRD

FORMAT                       GEOSOFT BINARY GRID

CONTENT                      RADAR

CREATION DATE                March 7, 2001

NUMBER of ROWS and COLUMNS   640 1024

PIVOTAL POSITION (X,Y)       331087 4212167

CELL SIZE                    83m

UNIT                         m

UNIT FACTOR                  0.01

MINIMUM VALUE (m)            157

MAXIMUM VALUE (m)            1217

MEAN VALUE (m)               536

PROJECTION INFORMATION :

DATUM                                 NAD27

SHEROID                               CLARKE 1866

PROJECTION                            UTM

CENTRAL MERIDIEN                      117 West

FALSE EASTING                         500000 m

FALSE NORTHING                        0 m

SCALE FACTOR                          0.9996