PROGRAM DOCUMENTATION

Program Name:		CENTRISET.EXE

Type:   			Main program

Purpose:  This program computes: 1) angular velocity or total time for settling specific particles from a given medium by centrifuge, and 2) settling velocity for gravitational settling analyses.

Operating System:		Windows 98/2000/XP

Source Language:		Just Basic (a subset of Liberty Basic)

Source Code: 		CENTRISET.EXE and associated forms

Program Category:	Data processing

Input:  For settling by centrifuge, the user must enter: fluid viscosity (poises), fluid density (g/cm3), particle density (g/cm3), particle radius (cm), time of acceleration (sec), time of deceleration (sec), initial distance from axis of rotation (cm) final distance from axis of rotation (cm), and either angular velocity (rev/sec) or total time (sec). To calculate settling velocity (cm/sec) for gravitational settling analyses, the user must enter the following five variables: fluid viscosity (poises), fluid density (g/cm3), particle density (g/cm3), particle radius (cm), and gravitational acceleration (cm/sec2).  Values for the viscosity and density of water at various temperatures and the density of common sedimentary materials are available through the information button.  The most commonly used numerical values for the acceleration of gravity are 980 cm/sec2 (the sea-level acceleration of free fall at a geodetic latitude of 45.5 degrees) and 979.7 cm/ sec2 (the average sea-level acceleration on Earth).

Maximum permissible precisions for these variables are:

Fluid Viscosity (poises) – 0.00001	Fluid Density (g/cm3) – 0.001

Particle Density (g/cm3) – 0.01		Particle Radius (cm) – 0.000001

Gravitational Acc. (cm/sec2) – 0.000	Angular Velocity (rev/sec) – 0.1

Total Time of Centrifugation (sec) – 0.1	Angular Velocity (rev/sec) – 0.1

Time of Acceleration (sec) – 0.1		Time of Deceleration (sec) – 0.1

Initial Distance From Axis of Rotation (cm) – 0.1

Final Distance From Axis of Rotation (cm) – 0.1

Particle diameters and radii at standard whole-phi intervals are:

Phi () Size		Diameter		Radius

mm		 m		   cm

_____________________________________________

4	    1/16	62.5		0.003125

5 	    1/32	31.25		0.001563

6 	    1/64	15.63		0.000781

7 	    1/128	  7.81		0.000391

8 	    1/256	  3.91		0.000195

9 	    1/512	  1.95		0.000098

10 	    1/1024	  0.98		0.000049

11 	    1/2048	  0.49		0.000025

Output:  Running the program generates the calculated variable in its standard units for the selected process and displays it in the results window.

Usage:  The program may be executed by simply selecting (i.e. double clicking on) the executable program CENTRISET.EXE. Alternately, the program may be executed by selecting START, then RUN, and then typing the path and name of the program.  The program opens a control window that presents the user with several options.  First, the user may simply enter the necessary variables for a centrifugation analysis calculation. Alternately, the user may select the Stokes Law button, which opens a new window and allows the user to enter the necessary variables for this process. The program will not run unless all necessary variables have been entered.

Selecting the "Calculate" command button executes the data processing and allows the user to view results in the corresponding results window and in a new calculation report window. The calculation report window records the entered variables and results of all runs conducted during a session and can be printed or saved through the file menu on the tool bar. Selecting the “Information” button allows the user to view: schematics of the equations, a textural grade scale, distance from axis of rotation calculations, look up tables for water density and viscosity and the densities of common materials, and this documentation. Alternately, selecting the “Clear all” button restarts the program.  Selecting “Exit” ends the program and closes the window.

Diagnostics:  If the times of acceleration and deceleration for too great for a given particle and medium, leaving no centrifuge run time, a “WARNING: Separation completed prior to expiration of total time!” will appear in the calculation report window.  If a necessary variable is left blank a “One or more required parameters unpopulated” error message will appear in the calculation report. If an unreasonable value is entered for a variable, an “Out of range” error message will appear for that variable in the calculation report.

Disclaimer:  Centriset is public-domain software developed by or for the USGS. Although these programs have been used by the USGS and Eliason Data Services, no warranty is made by the USGS or Eliason Data Services as to the accuracy and functioning of the programs and related program material. Distribution of these materials also does not constitute any warranty, and no responsibility is assumed by the USGS or Eliason Data Services in connection with any distribution activity.

Author:   A.H. Eliason, Eliason Data Services, 230 Meetinghouse Road, Mashpee, MA 02649, andrew@eliason.com; (508) 477-3155

Maintenance:   L. Poppe, USGS, 384 Woods Hole Road,

Woods Hole, MA 02543, lpoppe@usgs.gov; (508) 548-8700 x2314