GETTING STARTED

The file extraction program (EXTRACT) is designed to facilitate

the selection and copying of digital line graph (DLG) data files

from the CD-ROM.  The program is menu driven and runs on personal

computers with at least 512 Kbytes of free random access memory

(RAM) and an EGA or VGA video adaptor. The program is initiated by

selecting EXTRACT DLG DATA from the Main Menu.

The 1:100,000-scale DLG software may be executed from the CD-ROM

by typing DLG.BAT from the root directory on the CD-ROM.  The

software can also be installed on your hard disk.  To install

the software on your hard disk, choose menu option I from the

User Information submenu or type INSTALL from the root directory

on the CD-ROM.

The first screen that appears after selecting EXTRACT DLG DATA is

the main screen.  It contains a map of the State(s) for which DLG

data exist on the CD-ROM, a grid outlining the 1:100,000-scale

quadrangles, the title of the CD-ROM, a menu bar across the top of

the screen, and a command explanation box at the bottom of the

screen.  The software automatically checks for the presence of a

mouse.  If an activated mouse is detected, the mouse arrow prompt

appears in the middle of the screen.  If a mouse is not found, a

cross hair, which is controlled by using the arrow keys, appears.

The menu options across the top of the screen are HELP, SELECT

LAYERS, SELECT QUADS, VIEW QUAD, RESET, OUTPUT, and EXIT.  Options

that are not active remain shaded until a prerequisite selection

has been made.  For example, layers must be selected using the

SELECT LAYERS option prior to designating the quadrangle.  After

SELECT LAYERS has been used, the shaded area on SELECT QUADS is

removed and it becomes an active option.

HELP        Displays explanations of menu options.

SELECT      This option selects the layer(s) to be copied from the

LAYERS      CD-ROM.  The available layers include HYDROGRAPHY,

MISC. TRANS. (Miscellaneous Transportation), RAILROADS,

ROADS, and ALL LAYERS.  Use END SELECTION to exit the

SELECT LAYERS option.  The selected layer(s) will be

displayed on the screen for reference.  To change the

layer(s) after exiting from the SELECT LAYERS option,

use RESET to clear the current selection.

SELECT      This option is available after layers have been

QUADS       selected.  Choose SELECT QUADS, then place the mouse

arrow or cross hair inside the desired quadrangle.

Press either the left mouse button or enter key if

using the cross hair.  To cancel a selection, place

the mouse over the tagged quadrangle and press the

left button again.  As quadrangles are selected, each

quadrangle is tagged with a rectangle mark and the

amount of space required to store the data is displayed

on the screen.

VIEW QUAD   This option must be chosen prior to selecting layers.

It provides a view of an individual 1:100,000-scale

quadrangle that appears with a grid showing the

individual 15-minute sections within and adjacent to

the quadrangle.

After choosing VIEW QUAD, select the desired

quadrangle with the mouse.  The quadrangle outline

will be displayed along with the name of the

quadrangle and the southeast corner coordinates.  Use

SELECT LAYERS first, then use the mouse or cross hair

to select the 15-minute sections to be copied from

the CD-ROM.

OUTPUT      This option copies the selected layers for the chosen

quadrangles from the CD-ROM to the hard disk.  After

selecting OUTPUT, the user is prompted for the target

drive and subdirectory name.  If the subdirectory

does not exist, an option to create it is provided.

The amount of space required for the selected data is

then compared to the free space on the hard disk. If

there is not adequate space, an error message will

appear and the data will not be copied.

RESET       This option resets the program and clears selected

options to start over.

EXIT        This option returns the user to the Main Menu.

The software is designed to help the user identify and select the

data files for the layer(s) and area of interest to be copied from

the CD-ROM to the user's hard disk.  This can be done two ways:

1. Layer(s) for an entire 1:100,000-scale quadrangle(s) may be

selected.

2. Layer(s) for 15-minute cells within a quadrangle may be

individually selected.

The software does not allow a user to select both an entire

1:100,000-scale quadrangle and individual 15-minute cells within

the same session.  To accomplish both, the following steps must be

executed:

Session 1:

1) Select the EXTRACT DLG DATA option from the Main

Menu.

2) Choose SELECT LAYERS and designate the required ones.

3) Choose SELECT QUADS and enter the desired

quadrangle(s).

4) Choose OUTPUT to copy the data to the hard disk.

5) Choose EXIT or start another session from the main

screen.

Session 2:

1) Select the EXTRACT DLG DATA option from the Main

Menu.

2) Choose VIEW QUAD and select the desired quadrangle.

3) Choose SELECT LAYERS and highlight the layers needed.

4) Choose SELECT 15' CELLS and select the required

areas.

5) Choose OUTPUT to copy the data to the hard disk.

6) Choose EXIT from VIEW QUAD.

7) Choose EXIT or start another session from the main

screen.

The user can select HELP at any time between operations to obtain

further information and suggestions.  Also note that the user does

not have to return to the Main Menu to change selection methods.

After the data have been OUTPUT, the program resets and is ready

for a new session.

OUTPUT FILE FORMAT

The file naming convention for output files is as follows:

AABCCDEE

AA  = Two-character identification code for the 1:250,000-scale

quadrangle area of coverage for this file.

B   = Numeric code representing the quadrant of the 1:250,000-scale

quadrangle within which the area of coverage falls.

1=NW     2=NE

3=SW     4=SE

CC  = Indicates the data overlay category

RD = Roads

HY = Hydrography

RR = Railroads

MT = Miscellaneous Transportation

D   = Indicates cell size

S = 7.5-minute

F = 15-minute

EE  = Section/cell number within the 1:100,000-scale map

(see further explanation below)

Explanation of Section Numbering

The data for each overlay category for a typical 1:100,000-scale

quadrangle is divided into two 30-minute areas, the east and west

halves.  Each half is then further divided into four 15-minute

sections.  This sectioning scheme is diagrammed below.

WEST HALF          ||          EAST HALF

______________________________||_____________________________

|               |              ||              |              |

|               |              ||              |              |

|               |              ||              |              |

|    Section    |   Section    ||    Section   |    Section   |

|       1       |      2       ||       3      |       4      |

|               |              ||              |              |

|_______________|______________||______________|______________|

|               |              ||              |              |

|               |              ||              |              |

|               |              ||              |              |

|    Section    |    Section   ||    Section   |    Section   |

|       5       |       6      ||       7      |       8      |

|               |              ||              |              |

|_______________|______________||______________|______________|

||

SECTION NUMBERING SCHEME

Typical Quadrangle

There are some exceptions to this sectioning scheme.  If the feature

density in an area is great enough, the file size may exceed the

limitations of the processing programs used by the production

centers.  In this case, the 30-minute half is divided into sixteen

7.5-minute files.  Each half of the quadrangle is evaluated

independently.  For example, if the feature density in the west

half of a quadrangle exceeds the limitations, then only the west

half will be subdivided into sixteen 7.5-minute files.  The east

half will retain the typical sectioning.  This policy of assessing

the data in 30-minute halves results in three possible variations

of the typical sectioning scheme.  They are diagrammed below.

WEST HALF          ||          EAST HALF

______________________________||_____________________________

|       :       |      :       ||              |              |

|   1   :   2   |   3  :   4   ||              |              |

|       :       |      :       ||              |              |

|-------:-------|------:-------||       3      |       4      |

|       :       |      :       ||              |              |

|   9   :  10   |  11  :  12   ||              |              |

|_______:_______|______:_______||______________|______________|

|       :       |      :       ||              |              |

|  17   :  18   |  19  :  20   ||              |              |

|       :       |      :       ||              |              |

|-------:-------|------:-------||       7      |       8      |

|       :       |      :       ||              |              |

|  25   :  26   |  27  :  28   ||              |              |

|_______:_______|______:_______||______________|______________|

||

SECTION NUMBERING SCHEME

Quadrangle With a Dense West Half

WEST HALF          ||          EAST HALF

______________________________||_____________________________

|               |              ||       :      |      :       |

|               |              ||   5   :   6  |   7  :   8   |

|               |              ||       :      |      :       |

|       1       |       2      ||-------:------|------:-------|

|               |              ||       :      |      :       |

|               |              ||  13   :  14  |  15  :  16   |

|_______________|______________||_______:______|______:_______|

|               |              ||       :      |      :       |

|               |              ||  21   :  22  |  23  :  24   |

|               |              ||       :      |      :       |

|       5       |       6      ||-------:------|------:-------|

|               |              ||       :      |      :       |

|               |              ||  29   :  30  |  31  :  32   |

|_______________|______________||_______:______|______:_______|

||

SECTION NUMBERING SCHEME

Quadrangle With a Dense East Half

WEST HALF          ||          EAST HALF

______________________________||_____________________________

|       :       |      :       ||       :      |      :       |

|   1   :   2   |   3  :   4   ||   5   :   6  |   7  :   8   |

|       :       |      :       ||       :      |      :       |

|-------:-------|------:-------||-------:------|------:-------|

|       :       |      :       ||       :      |      :       |

|   9   :  10   |  11  :  12   ||  13   :  14  |  15  :  16   |

|_______:_______|______:_______||_______:______|______:_______|

|       :       |      :       ||       :      |      :       |

|   17  :  18   |  19  :  20   ||  21   :  22  |  23  :  24   |

|       :       |      :       ||       :      |      :       |

|-------:-------|------:-------||-------:------|------:-------|

|       :       |      :       ||       :      |      :       |

|  25   :  26   |  27  :  28   ||  29   :  30  |  31  :  32   |

|_______:_______|______:_______||_______:______|______:_______|

||

SECTION NUMBERING SCHEME

Quadrangle with High Feature Density Throughout

Additional information about the content and format of the

1:100,000-scale DLG files may be found in:

Digital Line Graphs from 1:100,000-Scale Maps.

(Data Users Guide 2)  U.S. Geological Survey, 1989.