UNITED STATES

DEPARTMENT OF THE INTERIOR

GEOLOGICAL SURVEY

WATER RESOURCES DIVISION

PEG TEST OF ENGINEER’S LEVEL

Date________________ Tested by _____________________________________

Level type and ID ___________________________________________________

c found ________	Fixed scale

Last test date_____________ 	c left __________ 	Peg test

TEST AS FOUND

|  | R | D |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |

(R1+R3) – (R2+R4 -**CR)

*c= 100  x ----------------------------------

(d2+d4) – (d1+d3)

(     +      ) – (      +         -         )

c=100 x   --------------------------------------

(             ) – (       +         )

-

c=100 x  --------------------------------------- =                   ___________________ As found

-

ADJUSTMENT (level remains set up at 2 and sighted at R4)

cd4

100

________                       100

Adjust cross hair to   R4  +/-            =                                                         =

Cross hair setting = ____________________

Normal form has information on repeating peg test and correction for curvature and refraction here !

| (d2+d4)/2 | **CR |
| --- | --- |
| 0 - 110 | 0 |
| 110 – 190 | 0.001 |
| 190 – 245 | 0.002 |
| 245 – 290 | 0.003 |
| 290 - 350 | 0.004 |

*c is the collimation factor, the inclination of the line of sight

in ft/100 ft, minus when up from the instrument, and plus

when down.

**CR is twice the curvature and refraction correction for a

sight of  (d2 + d4)/2  ft. Its value, which increases the rod reading

is tabulated at right.